NOTES:

Several people have asked questions about the new IDE driver.  Here are some

of the questions and answers:

1. Q. What drives does the IDE driver support?

A. Most IDE/ATA drives.

2. Q. Does the IDE driver support the IBM Model 35 and Model 40 in a file

server configuration?

A. Yes, if the system has the changes from IBM ECA# ECA087 implemented

(at least new FRU P/N 92F0140.)

3. Q. Does this driver fix the problem that ISADISK had with "user

definable" drive types?

A. Yes.  The ISADISK driver used the INT41 and INT46 vectors to find the

location of the drive or ROM parameter table.  ISADISK would abend if

the pointer to the table was below segment C800.  BIOSes which support

UD drive types typically put the table below C800:.  For example, AMI

and some other BIOS manufacturers use address 0300.  The IDE driver

queries the drives for drive information.  It does not use the ROM-

BIOS services.

4. Q. Given that the IDE driver obtains drive information from the drives,

will Ztest create problems?

A. No.  Since the drive parameters are located in sectors inaccessible

to a write command, Ztest cannot write over them.

5. Q. How should the user set up his IDE/ATA drives if he is using the IDE

driver?

A. A BIOS drive type is necessary in two cases: 1) a drive will have a

DOS partition or 2) a drive will be used to cold boot NetWare.  In the

second case, the user may select any type that is the same capacity

or nearly the same, but not more.  In the first case, the best

definition would be an exact definition.  However, a definition which

matches heads, sectors per track and has fewer than or equal to the

actual cylinders would suffice.

6. Q. Does the IDE driver support multiple paddleboards?

A. Yes, two paddleboards.  However, the paddleboards must be designed

with jumpers to configure IRQ and Base I/O (primary I/O 1F0h and IRQ

14, secondary I/O 170h and IRQ 11, 12 or 15).  The set-up would be

analogous to any other dual controller AT configuration.  The IDE

driver must be selected and configured for both channels.  It will

find all drives when it initializes.  The drive/s attached to the

secondary paddleboard will not be defined by the machine's CMOS.

7. Q. Does the IDE driver address the problem of data verification on the

extended AT bus (40 pin cable which connects the paddleboard or system

board to the IDE/ATA drives)?

A. Yes. In the VERIFY ON mode, it will read and compare data to ensure

integrity across the cables.  The cost is performance.  With verify

ON, performance will be degraded significantly.  Mirroring drives on

a single paddleboard will not improve integrity because the cable in

question is common to both drives.  With drives mirrored (duplexed)

on separate paddleboards, an intermediate level of data integrity

would be achieved without the speed penalty of setting verify ON.

8. Q. Will all paddleboards work in my system?

A. No.  It has been noted that some paddleboards will not work in some

machines, and some drives will not work with some paddleboards.  Refer

to your hardware manufacturer or system integrator for recommended

configurations.

The following limitations should be noted:

* The drive type number reported by the INSTALL program is generated by

the IDE disk driver at initialization time and is useful only to the

NetWare operating system.

* The IDE disk driver will not work with other ISA bus controllers (i.e.

ST506/412 and ESDI).  The IDE/ATA, ST506, and ESDI interfaces look

similar to the system software,  however, they are handled differently

by the disk driver.

* The VERIFY ON option uses significant processor resources.  It should

not be used with other processor-intensive options, such as mirroring

or Macintosh VAPs, due to possible adverse effects.

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************