HERCBIOS.ASM                   PAGE 1

THIS SYSTEM IS SHAREWARE, THEREFORE,

IF YOU LIKE THIS PROGRAM, FIND IT USEFUL, AND DECIDE TO USE IT,

THEN YOU MUST SEND A $5.00 LICENCE FEE  U.S. TO:

Douglas McDaniels

2627 Arlington Dr #101

Alexandria VA 22306 U.S.A.

Hercbios is Copywrite 1988 by Douglas McDaniels.

HOWEVER, HERCBIOS HAS BEEN RELEASED AS SHAREWARE, SO YOU MAY MAKE

ALL THE COPIES THAT YOU WANT, GIVE THEM TO YOUR FRIENDS, AND YOU MAY

PLACE THIS SYSTEM ON BULLETIN BOARDS.(HBIOSARC.EXE is the recomended

format)

First, what is HERCBIOS.COM ?

All the major video graphics boards for the IBM P.C. series  and

compatibles,  with  the  single  exception  of the Hercules Monochrome

Graphics Adapter, have a built in ROM BIOS (accessed though INT  10H)

that provides the video primitive functions such as the capability to

put a character on the screen. DOS uses these functions to  implement

its  teletype style interface, and these video ROM BIOS functions are

also accessed by many applications programs. There is no  reason  why

video  ROM  BIOS  functions  should  not be provided for the Hercules

Monochrome Graphics Adapter as well, and  that  is  what  HERCBIOS.COM

does. Using HERCBIOS.COM you can now have a 43 line by 90 column text

mode  for  your  M.G.A  (Hercules  compatable)   board,   and   those

application  programs  that  use  only  BIOS  calls for compatibility

should run under this mode.

You may be wondering what mode number I have decided to use for

the  Hercules board. All the IBM video modes have a number associated

with them, for example monochrome text mode is mode  number  7,  while

C.G.A text mode is mode number 3. The decision is quite easy. IBM has

started at 0, and has worked its  way  up  to  mode  20  or  so.  The

negative  numbers  have not been touched by anyone yet, so now is the

time to do so. I have  chosen  mode  -2  (or  254)  as  the  Hercules

monochrome graphics mode, reserving mode -1 for special use. Note that

this leaves room for a whole series of negative mode numbers for  use

by  other  non-IBM  board  manufacturers, and also for negative modes

that provide extensions to the INT 10H functions of already  existing

modes.  This program also implements a mode mapping feature, allowing

the user to redirect program requests for an IBM mode to a request to

change  to one of the negative, non-IBM modes (the special mode -1 is

involved in this feature, see the documentation  for  video  function

00h and the assembly listing for program MODEMAP )

__________________________________________________

HERCBIOS.ASM                   PAGE 2

Here  are  general user instructions

and general comments on the use of HERCBIOS.COM:

To install the program you simply run it, by entering  HERCBIOS

43, HERCBIOS 29, or HERCBIOS followed by some other number at the Dos

prompt.(The meaning of the command line argument is explained  later.

See  also the description of INT 10H function 08h) But to make use of

it you must also load the pixel maps. I have provided an accompanying

mini-program  ASCITABL.COM  which  loads the ASCII pixel bit maps for

IBM computers and true compatibles, and the Dos program  GRAFTABL.COM

loads  the  graphics  pixel  maps  for  those  with Dos 3.x. Just run

HERCBIOS.COM, ASCITABL.COM and GRAFTABL.COM in any order.  For  those

who  are  still  using  Dos  2.x,  I  have  provided a program called

GRAFCHAR.COM which duplicates the function of GRAFTABL.COM. Also, for

machines  which  are not really very IBM compatible, ASCITABL.COM may

not work. I have therefore provided an alternate program ASCICHAR.COM

which should work even on those machines, its disadvantage being that

it takes up more RAM than ASCITABL.COM since it must explicitly  load

the  pixel  maps, while ASCITABL.COM makes use of the pixel maps that

are supposed to be already present in the ROM BIOS.    A  word  about

mini-program  ASCITABL.COM,  this  program simply points INT 44H (the

interrupt reserved by IBM for just this purpose) to point to the  8x8

ASCII  pixel  maps,  which  on  IBM  and  true compatibles are always

located at  0F000:0FA6E,  and  I  have  hard  coded  this  vector  in

ASCITABL.COM.

To  change  into the Hercules mode you could use EGAMODE.COM (as

provided by the ZIFF DAVIS Publishing company) but you will  probably

want  to  use the mini-program CHANGE.COM (included) for this purpose

since it allows the user to decide whether  or  not  to  map  in  the

second  graphics  page,  leaving  it  out in order to provide for the

presence of a CGA card. In order  to  change  to  Hercules  monochrome

graphics  mode,  run  the  mini-program CHANGE.COM . If you provide a

command line argument of 0 (.i.e. type CHANGE.COM 0 ) then the second

graphics  page  (page 1) will be suppressed to allow a CGA compatible

card to reside in your system at  the  same  time  as  your  Hercules

compatible  card. To return to text only mode, in order, for example,

to run  programs  that  write  directly  to  the  screen  simply  run

TEXT.COM,  which  will  return  your  system  to  video  mode  7 (IBM

monochrome text only mode)

__________________________________________________

HERCBIOS.ASM                   PAGE 3

In order to select, at  the  Dos  prompt,  the  number  of  rows

displayed   on   the   screen,  I  have  provided  the  mini-programs

ROWS43.COM, ROWS29.COM, and ROWS25.COM.  Those  users  with  ANSI.SYS

installed  should  run  ROWS25.COM right off, since ANSI.SYS requires

that there be only 25 rows on the screen. For those who chose not  to

use  ANSI.SYS  (and  its  utility is very limited) running ROWS43.COM

will put 43 rows on the screen when in  Hercules  monochrome  graphics

mode,  and  running ROWS29.COM will return the display to the default

value of 29 rows on the screen. (Note that you will probably want  to

clear  the screen right after running these programs since previously

written rows are not altered)

A caution about CLS: The CLS  command  provided  with  Dos  will

suppress  the  alternate  graphics  page,  since when ANSI.SYS is not

installed, CLS clears the screen by issuing a change mode command  to

the  currently  active  video  mode.  Therefore,  I have provided two

clearscreen mini-programs  CLS0.COM  and  CLS1.COM  which  clear  the

screen, the latter permitting the existence of the alternate graphics

page, the former suppressing it.

Returning to the command line argument of HERCBIOS,  we  see  in

the  description of function 08H that it reserves enough RAM to store

the character codes for the read character function for the number of

rows  specified  in  the  argument.  No  space  is  reserved to store

character attributes (however, an algorithm is used  that  determines

if  the attribute is inverse video by examining the pixel patterns on

the screen. This algorithm works for all but a very few of  the  more

esoteric characters).

Note  that  line  drawing functions are included with these BIOS

functions.  I  have  included  the  Turbo  C  program  DAZZLE   which

demonstrates  the  efficiency  of  the line drawing routines. Try it!

(But be sure to use CHANGE to change into the new  bios  mode  before

running DAZZLE)

All names of products used here are trademarks or registered

trademarks of their respective owners.

This  brings  us  to the actual INT 10H AH functions themselves.

Here follows a writeup of all the supported functions, along with the

features specific to this implementation.

__________________________________________________

HERCBIOS.ASM                   PAGE 4

SET VIDEO MODE (Function 00H)

AX:AHAL CALL

BX:BHBL AH = 00H

CX:CHCL AL = Video mode to select

DX:DHDL BH = Number of highest graphics page to be mapped

into memory.(Currently for AL = 254 only)

SP CH = IBM mode number to map (AL =  255 only)

BP CL = Translation for future requests for mode in CH

SI      (AL = 255 only)

DI

DS RETURN

SS None

ES

Exceptions and Comments:

When  AL  =  255  a special mode mapping feature is invoked. Any

future requests for the mode contained in CH will be translated  into

a request for the mode number contained in CL. Also, the current mode

will be changed to mode 7 (monochrome text only)  as  a  side  effect.

Furthermore,    if   mode   254   is   placed   in   CL,   then   the

Get_Current_Display_Mode function  request  when  in  mode  254  will

return  the mode number in CL from then on, instead of 254. To undo a

mode mapping, simply use this function to remap the mode  to  itself.

It  would  probably  be undesirable to put 255 into CH or CL however,

unless you are quite sure that you know what you are doing.

______________________________________________________

SET CURSOR TYPE (Function 01H)

AX:AHAL CALL

BX:BHBL AH = 01H

CX:CHCL CH = Starting line within character cell for cursor

DX:DHDL CL = Ending line within character cell for cursor

SP

BP

SI

DI

DS RETURN

SS None

ES

__________________________________________________

HERCBIOS.ASM                   PAGE 5

Exceptions and Comments:

The default setting for mode 254 is CH = 06 and CL  =  07.  This

will  place  the  cursor at the bottom of the usual ASCII characters,

and two scan lines tall. For mode 254 the bottom of the  cursor  must

fit  within  the character cell, which is 12 scan lines if the number

of rows on the screen in less than 30, otherwise 8  scan  lines.  The

cursor does not blink.

____________________________

SET CURSOR POSITION (Function 02H)

AX:AHAL CALL

BX:BHBL AH = 02H

CX:CHCL BH = Page number

DX:DHDL DH = Row

DL = Column

SP

BP

SI

DI

DS RETURN

SS None

ES

Exceptions and Comments:

There  is a separate cursor associated with both page 0 and page

1. This function sets the cursor for the page contained in BH  (0  or

1).  For  mode  254, DH contains the row and may run from 0 to 42, or

from 0 to 28, depending on how many rows to  display  on  the  screen

have been selected. DL may run from 0 to 89 for mode 254.

_________________________________________________________

__________________________________________________

HERCBIOS.ASM                   PAGE 6

READ CURSOR POSITION (Function 03H)

AX:AHAL CALL

BX:BHBL AH = 03H

CX:CHCL BH = Page number (see function 02H)

DX:DHDL

SP RETURN

BP CH = Starting line for cursor

SI CL = Ending line for cursor

DI DH = Cursor's row

DL = Cursor's column

DS

SS

ES

______________________________________________

SET DISPLAYED PAGE (Function 05H)

AX:AHAL CALL

BX:BHBL AH = 05H

CX:CHCL AL = Page number

DX:DHDL

SP

BP

SI

DI

DS RETURN

SS None

ES

Exceptions and Comments:

For mode 254 there are two displayable pages, page 0 and page 1.

Users of this function should make sure that page 1 has  been  mapped

into memory before trying to make use of it.

____________________________________________

__________________________________________________

HERCBIOS.ASM                   PAGE 7

SCROLL UP (Function 06H)

AX:AHAL CALL

BX:BHBL AH = 06H

CX:CHCL BH = Attribute for blanked area

DX:DHDL CH = Top left row of window

CL = Top left column of window

SP DH = Bottom right row of window

BP DL = Bottom right column of window

SI AL = Lines to scroll

DI

DS RETURN

SS None

ES

Exceptions and Comments:

If  bits  0  to 2 are blank and either bit 4,5, or 6 is set then

the attribute is treated as inverse video, and a solid block is  used

as a fill character. If AL = 0, then the window is blanked.

___________________________________________________________

SCROLL DOWN (Function 07H)

AX:AHAL CALL

BX:BHBL AH = 07H

CX:CHCL BH = Attribute for blanked area

DX:DHDL CH = Top left row of window

CL = Top left column of window

SP DH = Bottom right row of window

BP DL = Bottom right column of window

SI AL = Lines to scroll

DI

DS RETURN

SS None

ES

__________________________________________________

HERCBIOS.ASM                   PAGE 8

Exceptions and Comments:

If  bits  0  to 2 are blank and either bit 4,5, or 6 is set then

the attribute is treated as inverse video, and a solid block is  used

as a fill character. If AL = 0, then the window is blanked.

___________________________________________________________

READ CHARACTER AND ATTRIBUTE AT CURSOR (Function 08H)

AX:AHAL CALL

BX:BHBL AH = 08H

CX:CHCL BH = Page (must be zero in any graphics mode,

DX:DHDL including mode 254)

SP RETURN

BP AH = Attribute (See note below) [***updated**]

SI AL = ASCII character code

DI

DS

SS

ES

Exceptions and Comments

When  installing  HERCBIOS, you may follow the program name with

an optional command line argument consisting of a number  from  0  to

43.  This  number  reserves  enough  RAM to store the ASCII character

codes for the number of rows that you specified starting from the top

down.  If  a  program  issues a get_char (function 08H) request for a

character in a row higher than you  specified  in  the  command  line

argument,  then  the  ASCII  value  for a space character is returned

instead. This function is used only by programs that  read  from  the

screen. It is usually apparent to the user that he is using a program

that does this, so if the user knows that he will not  be  using  any

programs  that  read  from  the screen in mode 254, then the user can

invoke HERCBIOS without any command line  argument,  which  will  not

reserve  any  space  for  the get_character buffer. This can save the

user up to 43*90 = 3870 bytes, which is almost 4Kb.  Also,  for  mode

254, xor'ed characters do not currently update the get_char buffer.

The  correct  normal/inverse  attribute will be returned through

the use of an algorithm that determins whether or not a character  is

normal  or  inverse  video  directly  from  the  screen  pixels.  The

algorithm returns the correct attribute for  all  except  characters,

177,178,219,220,  and  222,  and  in  43  row mode a few of the funny

characters in the range 0 to 31 are  also  misread.  This  should  be

sufficient for all but the most unusual screen reading requirements.

__________________________________________________

HERCBIOS.ASM                   PAGE 9

WRITE CHARACTER (USING ATTRIBUTE) AT CURSOR (Function 09H)

Note:  For  mode  254,  this  is  the  same function provided by

Function 0AH.

AX:AHAL CALL

BX:BHBL AH = 09H/(also AH = 0AH)

CX:CHCL AL = Character to write

DX:DHDL BH = Page

BL = Attribute

SP CX = Replication factor

BP

SI

DI

DS RETURN

SS None

ES

Exceptions and Comments

For mode 254, if bit 7 of BL  is  set,  the  character  will  be

xor'ed  into  the  display. If bits 0 to 2 of BL are blank and either

bit 4,5, or 6 of BL is set then the attribute is treated  as  inverse

video.  If  bit  3  of  BL  is  set,  and  bit 7 is not set, then the

character is Or'ed into the display. Xor'ed characters do not  update

the  get_char  buffer. Also, if more characters are written with a CX

greater than 1 than will fit on what remains of the  current  column,

then the result is unpredictable.

______________

WRITE PIXEL (Function 0CH)

AX:AHAL CALL

BX:BHBL AH = 0CH

CX:CHCL CX = Column (for mode 254: 0 to 719)

DX:DHDL DX = Row (for mode 254: 0 to 347)

AL = Pixel color (see below)

SP

BP RETURN

SI None

DI

DS

SS

ES

__________________________________________________

HERCBIOS.ASM                   PAGE 10

Exceptions and Comments

If bit 7 of AL is set, the pixel will be xor'ed into the screen.

For mode 254, the allowable colors are 0 and 1 for  black  and  green

(or  amber,  white ect.) If higher colors are used, additional pixels

will be placed on the  screen  near  the  selected  dot,  but  in  an

unspecified fashion. The currently displayed video page is used.

____________________________________________________

READ PIXEL ON SCREEN (Function 0DH)

AX:AHAL CALL

BX:BHBL AH = 0CH

CX:CHCL CX = Column (for mode 254: 0 to 719)

DX:DHDL DX = Row (for mode 254: 0 to 347)

SP

BP RETURN

SI AL = Pixel color (0 or 1 for mode 254)

DI

DS

SS

ES

WRITE CHARACTER IN TTY MODE (Function 0EH)

AX:AHAL CALL

BX:BHBL AH = 0EH

CX:CHCL AL = Character to write

DX:DHDL BH = Page

BL = Attribute

SP

BP

SI

DI

DS RETURN

SS None

ES

__________________________________________________

HERCBIOS.ASM                   PAGE 11

Exceptions and Comments

For  mode  254,  if  bit  7  of BL is set, the character will be

xor'ed into the display. If bits 0 to 2 of BL are  blank  and  either

bit  4,5,  or 6 of BL is set then the attribute is treated as inverse

video. Xor'ed characters do not update  the  get_char  buffer.  Bell,

backspace,  linefeed  and  carriage return characters are acted upon,

and the cursor is advanced appropriately. Linewrap is  also  provided

(90 characters per column)

_________________________________________

GET MODE (Function 0FH)

AX:AHAL CALL

BX:BHBL AH = 0FH

CX:CHCL

DX:DHDL

SP

BP

SI

DI

DS RETURN

SS AL = Current video mode

ES BH = Active video page

-----------------------------------------

GET ROWS (Function 20H/11H)

AX:AHAL CALL

BX:BHBL AH = 11H or 20H

CX:CHCL

DX:DHDL

SP RETURN

BP DL = Number of rows seen on screen

SI CX = Number of scan lines per character

DI       (Unique to non IBM video modes)

DS

SS

ES

__________________________________________________

HERCBIOS.ASM                   PAGE 12

-----------------------------------------

GET SIZE (Function 21H/19H)

AX:AHAL CALL

BX:BHBL AH = 19H or 21H

CX:CHCL

DX:DHDL

SP RETURN

BP DX = Number of pixel rows on screen

SI CX = Number of pixel columns on screen

DI      (unique to non IBM video modes)

DS

SS

ES

-----------------------------------------------------

SET NUMBER OF ROWS ON SCREEN (Function 22H/1AH)

AX:AHAL CALL

BX:BHBL AH = 1AH or 22H

CX:CHCL AL = Number of rows to set on screen (Unique to non

DX:DHDL      IBM video modes)

SP RETURN

BP None

SI

DI

DS

SS

ES

----------------------------------------------

MOVE TO POINT, PREPARE TO DRAW LINE (Function 23H)

(Unique to non IBM video modes)

AX:AHAL CALL

BX:BHBL AH = 23H

CX:CHCL AL = Color and style of line to be drawn, 0 = black

DX:DHDL     1 = green/white/amber ,129 = XOR line into screen

__________________________________________________

HERCBIOS.ASM                   PAGE 13

SP CX = X coordinate of start of line (0 to 719 for

BP Hercules and compatibles)

SI DX = Y coordinate of start of line (0 to 347 for

DI Hercules and compatables)

DS

SS RETURN

ES None

-------------------------------------------------------

DRAW LINE (Function 23H)

(Unique to non IBM video modes)

AX:AHAL CALL

BX:BHBL AH = 24H

CX:CHCL

DX:DHDL

SP CX = X coordinate of end of line (0 to 719 for

BP Hercules and compatibles)

SI DX = Y coordinate of end of line (0 to 347 for

DI Hercules and compatables)

DS

SS RETURN

ES None

Exceptions and Comments

The  start of the line is either set by a move command (function

23H), or else defaults to the  end  of  the  last  line  drawn,  thus

successive  draw  commands  can be used to draw polygons with no need

for any move commands for all but the first point of the  polygon  to

be  drawn. The style of the line (white, black, or XOR) is set by the

last issued move command until changed by another move command.

__________________________________________________

HERCBIOS.ASM                   PAGE 14

-------------------------------------------

Sincerely,

Douglas McDaniels

__________________________________________________