DEC 1994

NetWare Driver Installation

===========================

The NetWare driver programs on this diskette are for use with the D-Link

DE-650 PCMCIA Ethernet Card.

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory		    Contents

-----------------------     ---------------------------------------------------

\IBMPC\NETWARE		    A pre-generated IPXDE65N.COM.

\IBMPC\NETWARE\V215B	    NetWare v2.15b workstation drivers.

\IBMPC\NETWARE\V215C	    NetWare v2.15c, v2.2 or NetWare 386 workstation

drivers

\IBMPC\NETWARE\ODI.DOS	    ODI workstation driver for DOS.

\IBMPC\NETWARE\ODI.OS2      ODI workstation driver for OS/2.

\IBMPC\NETWARE\NET386	    Server driver for Netware v3.1x.

\IBMPC\NETWARE\NW40	    NetWare 4.0 server and ODI client drivers.

( support NetWare v 4.0x and v3.12 )

\IBMPC\NETWARE\TURBO.DRV    Turbo driver for NetWare 3.11 server and ODI client.

*********************************

*  QUICK SETUP FOR WORKSTATION	*

*********************************

A pre-generated workstation driver IPXDE65N.COM is included on this diskette.

You can simply run the IPXDE65N on a workstation to access the Novell network:

>IPXDE65N	  -- Supports NetWare Lite, 2.1x, 2.2 or NetWare 386.

>NETx	  -- This program is provided by Novell.

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXDE65N.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS LABEL command as follows:

>LABEL A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of this

diskette (or backup):

>Copy A:\IBMPC\NETWARE\V215C\*.*  B:\   -- For NetWare 386 workstation.

3)  Execute Novell's SHGEN or WSGEN program, which resides on the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-1 or WSGEN diskette.

The default values for all parameters of IPXDE65N.COM are as follows:

I/O port #1	 300

IRQ #1		 5

Note : If you generate IPX.COM by "1:Driver Configurable by JUMPERS

Utility", the I/O PORT # will be None, you must execute the JUMPERS

Utility offered by NetWare, the procedure is as follows:

a. >JUMPERS

b. Enter the file IPXDE65N.COM to modify.

c. The screen displays the following:

DE-650 Options

I/O PORT #1     None

d. Press Enter

(window assistance)

d1. I/O PORT#

=========

200

220

240

260

280

300

.

.

.

380

d2. Select one port#.

d3. Save the port# that you selected.

d4.	IRQ #1		 None

IRQ  #

======

2

3

4

.

.

15

d5. Select one IRQ#.

d6. Save the port# that you selected.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedure as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After the installation is completed, modify the STARTNET.BAT file as

follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------

DCD16

LSL

DE65NODI

IPXODI

CLIENT

DCD16.EXE

DCD16.EXE is an integration version of open adapter utility.

DCD16.EXE supports Intel 82365SL, Toshiba T3300SL's ICCNT, and Socket

Services v1.01, v2.0, v2.01, v2.10 Card Services v2.0, v2.01, v2.10.

For more detail information, please refer to \IBMPC\INSTALL\README.DOC.

3) Copy \IBMPC\NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may now run STARTNET to access the NetWare network.

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

D-Link ODI Driver program allows you to run multiple protocol stacks.  It also

allows you to install up to two network adapters in a workstation. D-Link ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.	For detailed info on Keywords,

please refer to Novell's manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the DE65NODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the DE65NODI.COM whenever a reference is made to the Hardware Specific

Module.

3)  A typical startup for ODI workstation is as follows:

>DCD16		-- Enable PCMCIA Adapter.

>LSL		-- Link Support Layer Module provided by Novell.

>DE65NODI		-- Hardware Specific Module provided by D-Link.

>IPXODI		-- IPX Protocol Stack Module provided by Novell.

>NETX		-- DOS Support Module provided by Novell.

>LOGIN

DCD16.EXE

DCD16.EXE is an integration version of open adapter utility.

DCD16.EXE supports Intel 82365SL, Toshiba T3300SL's ICCNT, and Socket

Services v1.01, v2.0, v2.01, v2.10 Card Services v2.0, v2.01, v2.10.

For more detail information, please refer to \IBMPC\INSTALL\README.DOC.

>DCD16 [/iop:P0] [/irq:P1] [/slt:P2]

Parameter description:

/iop   :I/O base

/irq   :IRQ value

/slt   :Slot number

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable irq value.

The default value is 0x5.

P2 :The selectable slot number.

The default value is 0. You must set 0 when you use slot 1.

Set slt:1 when slot 2 is used,etc.

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

Example :  NetWare 386 Server/Bridge Setup

===========================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\IBMPC\NETWARE\NET386\*.*  B:\    -- SYSTEM-1 disk must be in

drive B.

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load 386DE65N.LAN 	-- Select your network card settings.

:Bind ipx to 386DE65N

:Load install		-- Follow the NetWare installation two idential

LAN cards (bridge) instructions.

*********************************************

* NetWare 4.x SERVER and ODI CLIENT DRIVERS *

*********************************************

NetWare 4.x Server Setup

=========================

1. Install the NetWare 4.x CDROM properly.  Make sure that the PCMCIA

Card/Socket Services and Client Driver have been properly installed in

your notebook before installing the NetWare driver.	If you have any

questions about the PCMCIA software environment for your particular

notebook, please refer to the \IBMPC\INSTALL\README.DOC file on this

diskette.

2. Prepare a D-Link LAN driver diskette that includes the following files.

Insert D-Link driver diskette in A: driver.

Insert D-Link LAN driver diskette in B: driver.

1) Copy NWDE65N.LAN, NWDE65N.LDI from D-Link driver programs diskette

as follows:

copy A:\IBMPC\NETWARE\NW40\SRV\NWDE65N.LAN B:

copy A:\IBMPC\NETWARE\NW40\SRV\NWDE65N.LDI B:

2) Copy ETHERTSM.NLM, MSM.NLM from NETWARE 4.0 CD ROM or Diskette

copy L:\NETWARE.40\________\LANDRV\ETHERTSM.NLM B:

copy L:\NETWARE.40\________\LANDRV\MSM.NLM B:

3. Install NetWare 4.x SERVER.

L: (CD ROM drive)

L:\NETWARE.40\ENGLISH\INSTALL

4. Loading the LAN driver.

Choose a LAN driver that corresponds to the network interface board in

the server, repeat this step for any additional drivers.

1) Press <INS> to load a LAN driver not listed.

2) Press <F3> to specify a different path,

> B:\

( Put D-Link LAN driver into the diskette drive which contains

ETHERTSM.NLM and MSM.NLM )

3) Netware will display the list including the additional new driver.

Select the new LAN driver to install.

NWDE65N.LAN

D-Link DE-650 LAN Driver

4) Set driver NWDE65N parameters such as following :

Port value : 300H

Node address :

Frame type = Ethernet_802.2

Retries = 5

5) Bind the interface card to a protocol assign a unique IP Network address

to bind IPX to NWDE65N. (frame Ethernet_802.2)

6) Proceed and finish with installation.

ODI Client for DOS

===================

D-Link ODI Driver allows you to load multiple protocol stacks. It also allows

you to install up to two network adapters in a workstation. D-Link ODI driver

supports "port, node address and frame" options. You can use these options to

customize the NET.CFG file.  For detailed info on Keywords, please refer to

Novell manual "NetWare ODI shell for DOS - NET.CFG options".

Please install the NetWare ODI client driver files following the procedures

as described in the NetWare installation guide.

If you want to generate your own NetWare ODI client driver, please do the

following:

1)  Make a backup of WSDRV_1 diskette.

2)  Copy the appropriate DE65NODI.COM, DLKDRVS.DOS and DE65NODI.INS driver

files to DOS directory of WSDRV_1 diskette (or backup):

Insert D-Link driver diskette in A: driver.

Insert NetWare WSDOS_1 diskette in B: driver.

>Copy  A:\IBMPC\NETWARE\NW40\ODI.DOS\*.*  B:\DOS  -- For NetWare client

workstation.

Append contents of DLKDRVS.DOS to DRIVERS.DOS.

Contents of DLKDRVS.DOS is as follows:

" D-Link DE-100 Ethernet for ISA,DE100ODI.COM,DE100ODI.INS,IE0

D-Link DE-200 Ethernet for ISA,DE200ODI.COM,DE200ODI.INS,IE0

D-Link DE-220 Series Ethernet for ISA,DE220ODI.COM,DE220ODI.INS,IE0

D-Link DE-320 Ethernet for Micro Channel,DE320ODI.COM,DE320ODI.INS,00M

D-Link DE-400 Ethernet for EISA,DE400ODI.COM,DE400ODI.INS,0E0

D-Link DE-600 Ethernet Pocket LAN,DE600ODI.COM,DE600ODI.INS,IEM

D-Link DE-620 Ethernet Pocket LAN,DE620ODI.COM,DE620ODI.INS,IEM

D-Link DE-650 Ethernet PCMCIA,DE65NODI.COM,DE65NODI.INS,IEM           "

3)  Execute Novell's INSTALL program, which resides in the WSDOS_1 diskette.

When you are asked to install the driver for your network board, press

the Enter key.  You will be asked to insert a WSDRV_1 diskette.  Insert

this diskette (or backup) which contains DE65NODI.COM, DRIVERS.DOS and

DE65NODI.INS in drive A.

4)  Follow the NetWare Client Install normal installation procedures, Which

resides in the WSDOS_1 diskette. When asked to select a network driver,

simply choose "D-Link DE-650 PEMCIA Ethernet Card" for selecting

parameter like as Base I/O Port, Media Frame Type(s) and Optional Node

Address.

Upon completion, a new NET.CFG file will be created in your DOS directory.

A typical of NET.CFG is as follows:

Link Driver DE65NODI

PORT 300

FRAME Ethernet_802.2

:

NetWare DOS Requester

FIRST NETWORK DRIVE = F

:

5)  A typical startnet.bat for ODI workstation is as follows:

SET NWLANGUAGE=ENGLISH  -- Set NetWare 4.x to English language.

LSL		     -- Link Support Layer Module provided by Novell.

DE65NODI		     -- Hardware Specific Module provided by D-Link.

IPXODI		     -- IPX Protocol Stack Module provided by Novell.

VLM		     -- DOS Support Module provided by Novell.

F: 		     -- Change to connected device.

LOGIN .ADMIN.NV40SRV    -- Login to file server NV40SRV as user ADMIN.

6) You may run STARTNET to access the Novell network.