VIDTYPE 1.1

by Yousuf Khan

Introduction

============

You may use this program's executable, VIDTYPE.COM, with no obligations

or fees and include it in any software package, shareware or commercial

as you please. You may use all or parts of this program's source code,

VIDTYPE.ASM, with no fees, and with the sole condition that you

mention me within your own programming ventures.

This document divides itself between two sections. The first section

describes how to use this program and how to interpret its information.

The second section describes how it was programmed.

(1) How to use this program

===========================

IBM PCs were designed from the beginning to be able to use many

different types of video hardware. Not only that, but PCs can use upto

two different video subsystems at once. This program will identify many

of these adapters, and it will return configurable DOS errorlevels for

testing inside of batchfiles.

The following are the command-line parameters you can feed to the

program before running:

Param   Function

-----   --------

none    errorlevel based on prim subsystem standard video code

"e"     errorlevel based on prim subsystem extended video code

"2"     errorlevel based on sec subsystem standard video code

both    errorlevel based on sec subsystem extended video code

Examples would be:

vidtype

vidtype e

vidtype 2

vidtype e2

vidtype 2E

You'll note that upper or lower case do not matter.

The Standard or Extended video Code:

-----------------------------------

The standard video code returns standard IBM-recognized video adapters,

such as CGA, MDA, MCGA, EGA, or VGA. Just about all video adapters will

return at least one of these codes for compatibility.

The extended video code is returned only by extended adapter types like

the various Hercules or Super VGA adapters. The Hercules adapters, for

example, all appear to be standard MDA monochrome adapters to the

standard video code, but they are recognized separately in the extended

video code. Same for the Super VGA adapters, which all appear to be

standard VGA, but separate extended.

Extended MDA errorlevel codes:

0       no Hercules, regular MDA only

1       regular Hercules card

2       Hercules Plus card

3       Hercules InColor card

Extended VGA errorlevel codes:

0       regular VGA

1       ATI

2       Ahead

3       Paradise

4       Oak

5       Genoa

6       Cirrus

7       Chips&Tech

8       Tseng

9       ZyMos

If a VESA BIOS is present, then add 128 to errorlevels returned.

The Primary or Secondary Subsystem:

----------------------------------

The primary video subsystem is the system where DOS and BIOS will send

all their output to at present. The secondary is system is considered to

be dormant, but it to can be used independently by software that

recognizes the existence of two separate subsystems. At any time you can

swap which subsystem is primary or secondary with the DOS MODE command.

VIDTYPE will return a screen similar to the one below:

VIDTYPE v1.0 (c) 1992, Yousuf J. Khan

Primary video subsystem:

Standard video information:

VGA w/ color analog display

Extended video information:

Super VGA: ATI

VESA BIOS present

Secondary video subsystem:

Standard video information:

no display

Extended video information:

no extended info available

In the above example, we have a system with a single primary subsystem,

no secondary subsystem. It was determined to be a VGA standard, an ATI

Super VGA extended, and had a VESA BIOS installed.

(2) Programming highlights

==========================

You will find the complete assembler source code, VIDTYPE.ASM, included

in this archive. It was written in Masm-style code with Borland's Turbo

Assembler 2.0. Although I've attempted to maintain Masm compatibility as

best as I can, there's no guarantees that it will assemble under Masm.

There are four important instance variables in the system. They are

[Std0type], [Ext0type], [Std1type], and [Ext1type]. Each are a byte

long. [Std?type] variables all indicate an IBM standard video adapter

type, such as CGA, MDA, EGA, or VGA. [Ext?type] variables all indicate

extended information (if available) about those adapters, eg. whether an

MDA is also a Hercules-style monographics adapter, or whether a VGA is a

form of Super VGA. [*0type] variables indicate that the information has

been gathered for the primary video subsystem, while [*1type] indicates

info about the secondary subsystem.

There is a fifth instance variable, also a byte long, [egatype], which

is used only after the EGA adaptor detection test. Since the EGA can be

either a monochrome or color adaptor, after the test, this instance

variable indicates which one it was. If a monochrome EGA were found, for

example, then the MDA test could be be skipped later. Similarly if a

color EGA were found the CGA tests could be skipped.

The first test carried out is the procedure FindVGA. It uses video BIOS

function AX=1A00h, Get Display Combination Code, which is only available

on PS/2, MCGA, and/or VGA systems. If it is detected that this function

is supported by the current machine, then no more work needs to be done

to detect standard video display types, because all the relevant

information is returned in BX by this function. BH contains the display

combination code (DCC) for the secondary subsystem, and BL contains the

DCC for the primary. A list of DCCs is included inside the FindVGA

procedure.

However, even though the DCC is returned, it only represents information

relevant to standard video adapter types, tests still need to be done to

determine extended video types. If an MDA is found then the FindHGC

procedure performed afterwards, to determine if it's Hercules. If a VGA

is found then the FindSVGA procedure is performed.

The FindSVGA procedure will determine if a VGA has a Super VGA

extension. There are currently 9 distinct SVGA types that it will

identify. Some of the SVGAs can be identified easily by looking for

certain strings at prespecified locations on their BIOS. Other SVGAs

can only be identified by testing for the existence of extended,

brand-specific registers. Whatever the case, it will return a value

between 0 and 9 which will be placed in one of the [ext?type] instance

variables. Later on, FindSVGA will also determine if the BIOS conforms

to the VESA BIOS specfications. If a VESA compliant BIOS is found then

the 7th bit of [ext?type] will be set, in other words increasing the

value of [ext?type] by 128.

The FindEGA procedure uses a video BIOS function available only on EGA

and later adapters, Int 10h, AH=12h, BL=10h, GET EGA INFO. If this

function is supported then BX will have been changed upon return. BH

will return the fact that the EGA is either in monochrome or color mode,

which is then be used to set the [egatype] instance variable. No

[ext?type] instance variables are set during this function.

The FindEGA procedure is performed next. If the [egatype] instance varb

indicates that a color EGA was found, then this procedure is bypassed.

Otherwise it determines if a CGA is installed by testing for the

presence of 6845 or compatible CRTC at the assumed i/o port location of

color adapters, 3D4h.

The FindMono procedure is performed last. If the [egatype] indicates a

mono EGA was found, then this procedure is skipped. Otherwise the same

6845 detection procedure is performed as with the CGA, but this time at

port location 3B4h. And if it is determined that an MDA adapter is

present, then an extended test for Hercules, FindHGC, is called.

The FindHGC procedure is called only after it has been determined that

an MDA exists. It returns a value from 0 to 3 depending whether and what

type of Hercules or compatible card is installed.

The Find6845 procedure is used by both FindCGA and FindMono to test for

the presence of a 6845 at their respective assumed locations.

The Determine_subsystem procedure is used only by FindEGA, FindCGA, and

FindMono. It will make a determination whether the currently tested

subsystem is the primary or secondary subsystem. It makes this

determination by looking at offset 63h in the BIOS Data Segment (ie.

segment 40h), which contains the value of video base port address. That

is if the color subsystem is active then it will contain 3D4h. If the

mono is active then it will contain 3B4h.

The Write_msg procedure displays all this information to the user.

The Return_errorlevel procedure parses the command-line and returns a

DOS errorlevel. The only possible command-line parameters are "2" and

"e". If "e" (case-insensitive) is present then the errorlevel returned

will be based on the [ext0type] value. If "2" is on the command-line,

then the errorlevel is based on the [std1type]. If both parameters are

present, then the errorlevel returned is from [ext1type]. If no

parameters are on the command-line, or garbage parameters are on the

command-line then the errorlevel is based on the [std0type] value.*

* Note: there is a bug in the command-line parsing here and simply

sending no parameters will not work, you must send garbage parameters.

Version History

===============

Vers #  Date    Description

------  ----    -----------

1.0     Nov 92  -first release to public

1.1     Nov 92  -fixed a silly bug that made a Tseng mistakenly send the

return code for a C&T, and made a ZyMos send the return

code for a Tseng. Pointed out by John Olson in Fidonet's

80XXX echo.

-added Trident chipset detector sent over by the same

John Olson, above.

-added an installation check for the 8514/A's

HDILOAD.EXE driver. Treated like any other Super VGA

installation check.

-received reports that the C&T chipset test was causing

lockups on other people's machines. Have moved test

location to the very last position, so hopefully other

chipsets have a chance to be detected before this test

is performed. May consider commenting over this test

entirely, if any more reports come in about lockups.

-command-line bug is still present.

Conclusion

==========

Well, that's that. I hope you find this program useful. Better yet, I

hope you find this program educational, that's why I included its source

code. If you have any questions, or if you got any hot new Super VGA

detectors that you would want me to implement, write to me at:

Postal address: Yousuf J. Khan

1007 Alenmede Cres.

Ottawa, Ont., Canada

K2B 8H2

Fidonet address: 1:163/215.6

Internet address: 1:163/215.6@fidonet.org