I/O Ports and Controllers on IBM Compatibles and PS/2

Compiled	and Annotated by

William C. Parke

(c) 1985,'86,'87,'88

Send comments	or corrections to:

William C. Parke

1820 S Street	NW

Washington, D.C.	20009

----------------------------------------------------------------------------

The following gives the	input and output ports used by IBM PC compatible

computers.  Most ports allow access to input and output	device

controllers.  A	controller may have several internal registers which can

be set to fix the operation of that controller,	read its status, or send

data to	it.  Other ports access	discrete logic latches or registers.

Some ports are write-only or read-only.	 It is even possible for a port

to be write-only and read-only by using	two different registers	at the

same port address.  Port addresses can range from 0 to 0FFFFH (64K

ports).	However, PC system boards generally reserve only the first 1024

(0-3FFH). The AT and PS/2 freely employ	ports above 3FFH for extended

applications, such as the two port asynchronous	adaptors and network

adaptors.

Ports from 0-0FFH have a special significance, since the 80x86 can use

byte rather than word port addresses in	the IN and OUT instructions.

----------------------------------------------------------------------------

Port	  Function and Chip Server (if present)

----------------------------------------------------------------------------

000-00F  DMA Direct Memory Access Processor 8237A-5 or	equivalent

010-01F  Extended DMA on PS/2 60-80

020-021  Interrupt Generation 8259A Master Interrupt Processor

030-03F  AT 8259 Master Interrupt Controller extended ports

040	  System Timer 8253  Count Reg,	Channel	0 System timer

042	  Count	Reg, Channel 2 Speaker audio

043	  Control byte for channels 0 or 2

044	  Extended Timer for PS/2 Count	Reg, Channel 3 Watchdog	timer

047	  Control byte for channel 3

050-05F  AT 8254 Timer

060-063  PPI Status Port PPI 8255 Keyboard, PC	read switches SW1, SW2

064	  Extended PPI PS/2 8042 Keyboard, Aux.	Device Controller

070-071  AT and PS/2 CMOS ports RTC/CMOS MC146818 Clock and PS/2 NMI mask

081-083  DMA Controller Registers 0-2

087	  DMA Controller Register 3

089-08B  DMA Controller Registers 4-6

08F	  DMA Controller Register 7

090	  Central Arbitration Control Port PS/2

091	  Card Selection Feedback PS/2

092	  System Control Port A	on PS/2: Control and Status Register

094	  Video	System Setup on	PS/2: Programmable Option Select (POS) Setup

095	  PS/2 Reserved

096	  POS Channel Select on	PS/2: Program Option Select Adapter

0A0	  PC NMI Mask Register on PC: Discrete latch for masking the NMI

0A0-0A1  Slave	Interrupts on AT, PS/2 8259A Slave Interrupt Controller

0B0-0BF  AT 8259 Slave	Interrupt Controller extended ports

0C0-0DF  DMA Controller on PS/2 or Sound Generator on PCjr (0C0 port)

0E0-0EF  AT reserved

0F0-0FF  Math Coprocessor 80x87 on PS/2 or PCjr diskette controller

100-107  Program Option Select	on PS/2: POS Registers

110-1EF  AT I/O options

1F0-1F8  AT Fixed Disk	Controller

200-20F  Game I/O Ports, discrete logic (Active port: 201H)

210-217  PC Expansion Unit

21F	  AT reserved

248-24F  Serial #8 option 8250	ACE

268-26F  Serial #6 option 8250	ACE

278-27F  Parallel Port	LPT3: discrete logic

2A2-2A3  Clock	option:	MSM58321RS Clock Chip

2B0-2DF  Alternate EGA	on PC and AT

2E1	  AT GPIB Adapter 0

2E2-2E3  AT Data Acquisition adapter 0

2E8-2EF  Serial Port COM4: on PC 8250 ACE used	with IRQ3

2F8-2FF  Serial Port COM2: on PC and PS/2 ACE used with IRQ3

300-31F  PC Prototype card on PC

320-32F  XT Fixed disk	Controller

348-34F  Serial #7 option 8250	ACE

368-36F  Serial #5 option 8250	ACE

378-37F  Parallel Port	LPT2: discrete logic parallel port

380-38C  SBSI on PC, AT Secondary Binary Synchronous Interface

3A0-3A9  PBSI on PC, AT Primary Binary	Synchronous Interface

3B4	  Monochrome Index Reg.	  6845 Video Controller	CRT Index Register

3B5	  Monochrome Control Regs.6845 Video CRT Control Registers

3B8	  Mono.	Control	Port	  6845 CRT Control Port

3BA	  Mono.	Status/Feature	  Input	Status/Output Feature Control Reg.

3BC-3BE  Parallel Port	LPT1: discrete logic parallel port using IRQ7

3C0-3C5  Video	Subsystem	  VGA Attribute	and Sequencer Registers

3C6-3C8  Video	DAC		  VGA Digital-to-Analog	Converter

3CE-3CF  Video	Subsystem	  VGA Graphics Registers

3D4	  PC Color Graphics	  6845 Video Controller	CRT Index Register

3D5	  PC Color Graphics	  6845 Video CRT Control Registers

3DA	  PC Color Status/Feature Input	Status/Output Feature Control Reg.

3E8-3EF  Serial Port COM3:on PC 8250 ACE used with IRQ3

3F0-3F7  Diskette Controller 765 Chip

3F8-3FF  Serial Port COM1: on	PC and PS/2 ACE	used with IRQ4

6E2-6E3  AT Data Acquisition Adapter 1

790-793  Cluster Adapter 1

AE2-AE3  AT Data Acquisition Adapter 2

B90-B93  Cluster Adapter 2

EE2-EE3  AT Data Acquisition Adapter 3

B90-B93  Cluster Adapter 3

22E1	  AT GPIB Adapter 1

2390-2393 Cluster Adapter 4

42E1	  AT GPIB Adapter 2

3220	  Serial Port COM3:   PS/2 Asychronous Adaptor

3228	  Serial Port COM4:   PS/2 Asychronous Adaptor

4220	  Serial Port COM5:   PS/2 Asychronous Adaptor

4228	  Serial Port COM6:   PS/2 Asychronous Adaptor

42E1	  AT GPIB Adapter 2

5220	  Serial Port COM7:   PS/2 Asychronous Adaptor

5228	  Serial Port COM8:   PS/2 Asychronous Adaptor

62E1	  AT GPIB Adapter 3

82E1	  AT GPIB Adapter 4

A2E1	  AT GPIB Adapter 5

C2E1	  AT GPIB Adapter 6

E2E1	  AT GPIB Adapter 7

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 00-0FH and 81H-83H: The 8237A DMA	Controller Chip	and PS/2 Extensions

----------------------------------------------------------------------------

The 8237A chip controls	direct memory access (DMA) to and from a set of

16 ports from 000H to 00FH.  There are three DMA channels available. DMA

is performed by	stealing a CPU bus cycle, setting a wait state if

necessary, to transfer each byte.  Ports 081H to 083H are DMA page

register ports for DMA channel 1-3.  Amoung the	20 bits	for memory

addressing during DMA transfer,	the first four bits are	the output of

the page register.  The	last 16	bits are output	from the 8237A.

On an IBM PC compatible, channel 0 is used by the hardware to refresh

dynamic	RAM.  This channel is not available to software.

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 20H-21H: Master 8259A Interrupt Controller

-------------------------------------------------------------------------

The first 8259A	occupies two port addresses at 020H-021H from which four

initialization command word registers and three	operation command word

registers are set. Three interrupt status registers can	be read.

The 8259A can be used to generate INTR 80x86 type interrupts from a

hardware device.  Moreover, several 8259As can be chained together.  The

ATs and	PS/2s use a second slave 8259A.

The 8259A has three one-byte registers to control and monitor eight

hardware interrupt lines IRx.  A bit in	the interrupt-request-register

(IRR) is set when an interrupt request line becomes active.  The

in-service-register (ISR) is checked to	see if another interrupt is in

progress.  Also, the priority of the interrupts	is checked.  Then, the

interrupt-mask-register	(IMR) is used to verify	if the interrupt is

allowed.  The IMR can be programmed to mask interrupts by setting the

corresponding bits and sending the byte	to the second controller port.

After a	hardware interrupt service routine is finished,	the controller

interrupts must	be re-enabled by sending 20H to	the first controller

port.

On IBM PC machines, the 8259A is set	to respond to positive-going

edges on the interrupt request lines (IRQ0-IRQ7), to use interrupt

vectors	08-0FH,	to use buffered	mode, and to re-initialize interrupts

upon receipt of	020H (end-of-interrupt)	code on	port 20H.  Configuration

can be performed using:

MOV AL,13H	   ; edge-triggered, one 8259a,	icw4 needed

OUT 20H,AL

MOV AL,8	   ; use interrupt vectors 08-0fh for IR0-IR7

OUT 21H,AL

MOV AL,9	   ; icw4 buffered mode, normal	eoi, 8086 CPU

OUT 21H,AL

To enable BIOS interrupt routines, use mask 0bch (0 bit	means enable):

MOV AL,0BCH	   ; enable disk (bit 6), keyboard (bit	1), timer (bit 0)

OUT 21H,AL

STI

MOV AL,20H	   ; eoi command

OUT 20H,AL

The IRQn lines are set to CPU interrupt	service	by the master and slave

interrupt controllers in the following order:

Hardware Interrupts			     Software Service Routine

IRQ0 Timer every 0.054897095 seconds		INT  8H

IRQ1 Keyboard	interrupt service		INT  9H

IRQ2 I/O channel, slave 8259,	vga		INT 0AH

IRQ8  Real Time Clock			      INT 70H

IRQ9  Replace IRQ2 and LAN Adapter interrupt    INT 71H

IRQ10 Reserved				      INT 72H

IRQ11 Reserved				      INT 73H

IRQ12 Mouse interrupt			      INT 74H

IRQ13 Math coprocessor			      INT 75H

IRQ14 Fixed disk controller		      INT 76H

IRQ15 Reserved				      INT 77H

IRQ3 Serial device COM2			  INT 0BH

IRQ4 Serial device COM1			  INT 0CH

IRQ5 Hard drive int.(also LPT2 on AT)		  INT 0DH

IRQ6 Diskette	drive interrupt			  INT 0EH

IRQ7 Parallel	port device LPT1		  INT 0FH

General	programming considerations:

General	initialization sequence	(* for IBM PC):

1. Send	initialization command "word" (ICW1) to	port 20H as byte:

bit	   description

7-5	   A7-A5 vector	address	table for 8080 or 8085 sytem

* 000 if 8086 system

4	 * 1  ICW1 identifier bit

3	 * 0= edge sensitive interrupt

1= level sensitive interrupt

2	   0= eight byte vector	addresses in interrupt table

* 1= four byte	vector addresses in interrupt table (IBM)

1	   0= Several 8259A chips in system (cascade mode, ICW3	needed)

* 1= only one 8259A chip in system (no	slaves,	no ICW3	needed)

0	   0= ICW4 not needed

* 1= ICW4 to be sent

2. Send	ICW2 to	port 21H as byte:

bit	   description

7-3   A15-A11 if 8080/85 system,

* A7-A3 vector	address	if 8086	system

0-2   A10-A8 if 8080/85 system

* 000 if 8086 system

3. If ICW1 bit 1=0, send ICW3 to 21H as	mask to	show which IRn lines have

slave 8259As.  If this 8259A	is a slave, use	bits 0-3 to set	value

of master IRn to which this slave is	attached.

4. Send	ICW4 to	port 21H as byte:

bit	   description

7-5	   not used, set 0

4	 * 0= serve interrupts sequentially

1= priority nested order: IR0 > IR1 > ... IR7

3	   0= non-buffered mode, ignore	bit 2

* 1= buffered mode selected, check bit	2 and use SP/EN	line

2	 * 0= slave if bit 3=1

1= master controller	if bit 3=1

1	 * 0= not automatic end-of-interrupt (must send	eoi=20h	to 21h)

1= automatic	end-of-interrupt

0	   0=8080/8085 mode

* 1=8086 mode

Commands:  The following operation command "words" can be sent to the

8259A (in any order, as	needed), by out	commands to port 20H and 21H:

Operation Command "Word" 1: Mask for IR0-IR7 (reset bit	to 0 to	enable

interrupt); send to 21H.

Operation Command "Word" 2 to port 20H:

bit	   description

7	   0= no rotation of interrupt priority

1= rotate interrupt priority	according to:

bits 6 5 = 1 1 current IRn (bits 2-0) set	to lowest priority

0 1 ignore bits 2-0

0 0 no	EOI command will be issued

6	   0= disable bits 0-2

1= enable bits 0-2

5	   0= do not issue EOI command to CPU

1= issue EOI	to CPU

4-3	   0 0	OCW2 identifier

2-0	   index of IRn	for this command

Operation Command "Word" 3 to port 20H:

bit	   description

7	   not used

6	   0= ignore bit 5

1= honor bit	5

5	   0= disable special mask mode

1= enable special mask mode

4-3	   0 1 OCW3 identifier

2	   0= poll command has not been	issued

1= override bit 1 and poll command has been issued

1	   0= no read register command issued

1= read register command issued

0	   0= interrupt-request	register will be read by read-status operation

1= in-service register will be read by read-status operation

Interrupt status read:

in al,021h	; gives	contents of interrupt-mask register

mov ah,al

in al,020h	; gives	contents of in-service register	or

;  interrupt-request register, according to command

;   "word" 3 bit 0

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 40H-43H: The 8253	Timer Chip

----------------------------------------------------------------------------

Ports 44H,47H: PS/2 Watchdog Timer Counter and Control Port

----------------------------------------------------------------------------

The 8253 is a programmable three-channel 16-bit	interval timer/counter,

occupying four ports from 040H to 043H.	 Each channel can be used to

take an	input clock signal 0-2MHz and produce an output	signal by

dividing by an arbitrary 16-bit	number.	 Channel 0 is used to make the

time-of-day clock ticks, channel 1 is used to tell the DMA to refresh

the dynamic RAM, and channel 2 is used to make a audio signal for the

speaker.  Each channel can be programmed in one	of six modes: 0=

interrupt on terminal count, 1=	programmable one-shot, 2= rate

generator, 3= square-wave generator, 4=	software-triggered strobe, 5=

hardware-triggered strobe.

Initialization:	 Send a	mode control byte for each channel to the 8253

control	word register at 043H.	Send a count to	timer port, one	byte at

a time.

Mode Control byte sent to the Control register for Channels 0, 2

bit	 description

7-6	 counter number	(0-2)

5-4	 latch read format

00= latch current count for reading

01= read/load high byte (no latching	needed)

10= read/load low byte (no latching needed)

11= read/load low, then high	byte

3-1	 mode number

000=interrupt on terminal count

001=programmable one-shot

010=rate generator

011=square wave generator

100=software	triggered strobe

101=hardware	triggered strobe

0	 0= count in binary

1= count in BCD

Mode Control byte sent to the Control register of Channel 3:

bit	 description

7-6	 00 = select counter 3

01 = R/W counter bits 0-7 only

3-0	 reserved, set 0

Counter	Latch Command sent to the Control register of Channel 3:

bit	Function

7	SC1 - specifies	counter	to be latched

6	SC0 - specifies	counter	to be latched

5-4	00 = counter latch command

3-0	reserved, set 0

Port assignments	     Power-up mode byte	 Power-up count	on PC

040H  timer 0 TOD clock	     036H mode 3	 0=65536   (18.2Hz)

041H  timer 1 DMA refresh    054H mode 2	 12H=18	   (66 kHz)

042H  timer 2 Speaker tone   0B6H mode 3	 553H=1331 (896	Hz sq.wave)

043H  control word register

for channels 0-2

044H  counter 3

047H  control word register

for channel 3

Channel	0: The System Timer:

The TOD	clock is set by	the BIOS to pulse every	18.2 times a second.  On

each pulse, Int	8 is generated.	 The Int 8 service routine keeps a tally

at the double word at 40:6Ch.  The channel 0 system timer latch	can be

cleared	by a system reset, by the Int 8	service, or a write to port 61h

with bit 7=1.  Disk timing operations are also controlled bye this

service.

Channel	1: DMA Refresh Pulses:

The DMA	refresh	pulse causes the DMA chip to refresh all RAM.  This

channel	should not be reprogrammed.

Channel	2: Tone	Generation for Speaker:

The Speaker tone timer is connected to the computer speaker. The gate to

this timer are controlled by the 8255 interface	chip.  The gate	is

closed by sending bit 0	to 1 at	port 61H.  The output to the speaker can

be close with bit 5 at port 62H.

An IN instruction at port 43H will place the data buffer in the	high-

impedence state	with no	further	operation.  The	control	word bit pattern

is:

bits	 7-6 Number of channel to program

5-4 Kind of operation

00 = move	counter	value into latch

01 = read/write high byte	only

10 = read/write low byte only

11 = read/write high byte, then low byte

3-1 Mode number (0-5)

0 If	0, binary data,	else BCD

After the control word is sent OUT to port 43H,	set channel 2 to enable

clock signal (bit 0) at	port 60H.  Use 1 to drive speaker, 0 for timing

operations.  Send counter LSB to 42H, then MSB.

Channel	3: The Watchdog	Timer (PS/2 only):

The watchdog timer and system channel time-out are not masked by sending

an 80H to port 70H.  The watchdog timer	detects	when IRQ0 is active for

more than one clock period.  If	so, its	counter	is decremented.	 When

the count reaches 0, a NMI is generated.  Thus,	if the IRQ0 is not being

serviced, an error can be detected. When the watchdog timer sets a NMI,

then it	also sets I/O 094h bit 4.  NMI stops arbitration until 090h bit

6 = 0.

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 60H-63H: PC 8255 Parallel	I/O Port Chip for Keyboard and Status

----------------------------------------------------------------------------

Ports 60H-64H: PS/2 Intel 8042 Keyboard/Auxiliary Device Controller

----------------------------------------------------------------------------

The 8255 Parallel Port Controller and Programmable Peripheral Interface

The 8255 chips control parallel	ports on the PC	system,	and acts as the

Programmable Peripheral	Interface (PPI)	for the	CPU, occupying four

consecutive port addresses 060H-063H.  The PPI can control three

independent ports (A, B, and C)	as either input	or output.  The	fourth

port address is	used as	a control port for the chip.  The following

shows the meaning of a control byte sent to the	write-only control port:

bit			  Value	     Action

7   Mode Set	Flag	      0	 Inactive   1  Active

6,5   Mode Selection A	     00	 Mode 0	   01  Mode 1	1x  Mode 2

4   Port A		      0	 Output	    1  Input

3   Port C (upper 4 bits)  0	 Output	    1  Input

2   Mode Selection B	      0	 Mode 0	    1  Mode 1

1   Port B		      0	 Output	    1  Input

0   Port C (lower 4 bits)  0	 Output	    1  Input

If bit 7 is 0, the byte	sent is	used to	set or reset a bit in port C.

Mode 1 uses three port C lines for handshaking and interrupt control of

port A.	 For input, if PC4=0, port A latches data and PC5 goes high to

indicate 'buffer full' for device connected to input lines.  PC5 returns

low when the CPU reads port A.	If port	A interrupts are enabled, PC3

also goes high when a byte is received,	which can be used for an IRn

line to	an 8259A interrupt controller.	Port B functions like port A in

mode 1 except it uses the three	low bits of port C for control.	 Output

in mode	1 is similar.  Mode 2 allows port A to operated	bidirectionally,

with handshaking and interrupt control using five bits of port C.

The PPI	Status Ports on	the IBM	PC compatibles at port addresses 060H to

062H perform the following functions (all set to mode 0, A made	input, B

output,	C input	by sending 099H	to 063H):

060H Port A Input (acts	as a one byte device output register):

If PB7 = 0 Read	Keyboard Scan Code

If PB7 = 1 Read	switches

PA7,6   = SW1-8,7  # of drives

PA5,4   = SW1-6,5  monitor type

11 = monochrome

10 = 80x25 color

01 = 40x25 color

PA3,2,0 = SW1-4,3,1 Reserved

PA1     = SW3	   Math	chip mounted

061H Port B Output (acts as a one byte device control register):

PB7 0 enable keyboard read

1 clear keyboard and	enable sense of	SW1

PB6 0 hold keyboard clock low, no shift reg. shifts

1 enable keyboard clock signal

PB5 0 enable i/o	check

1 disable i/o check

PB4 0 enable r/w	memory parity check

1 disable r/w parity	check

PB3 0 turn off LED

1 turn on LED (old cassettee	motor off)

PB2 0 read spare	key

1 read r/w memory size (from	Port C)

PB1 0 turn off speaker

1 enable speaker data

PB0 0 turn off timer 2

1 turn on timer 2, gate speaker with	square wave

062H Port C Input (acts	as a one byte device output register):

(Set PB2 (PC) or PB3 (XT) first.)

PC7 0 no	parity error or	PB4=1

1 r/w memory	parity check error

PC6 0 no	i/o channel error or PB5=1

1 i/o channel check error

PC5 0 timer 2 output 0

1 timer 2 output 1

PC4   reserved (old cassettee data input)

PC3,2,1,0 = r/w memory (SW2-4,3,2,1) if PB2=1

=  spare key (SW2-8,7,6,5) if PB2=0

PC7 and PC6 are used by the NMI handler to tell whether RAM parity

error, i/o channel status error,	or, if both are	0, an 8087 error

occured.

Example: Direct	reading	of PC keyboard scan code (replacement for INT 09):

1. Read scan code.  Note that "make" key scan code has	bit 7=1,

"break" code has bit 7=0, except on	AT, for	which bit 7 is always 0,

a "break" produces a 0F0H code, then the key scan code.

2. Send acknowledge to	keyboard by toggling bit 7 to 1, then back to 0.

3. Put	keyboard in buffer.

4. Signal EOI to the interrupt	controller.

pushall

in al,060h     ;	get key	code

push ax	      ;	save it

in al,061h     ;	get current control

mov ah,al      ;	save PB	control

or al,80h      ;	set keyboard bit

out 061h,al    ;	keyboard acknowledge

xchg ah,al     ;	get back PB

out 061h,al    ;	reset PB control

pop ax	      ;	get back code

...	      ;	save code in buffer

cli

mov al,20h

out 20h,al     ;	send eoi to interrupt controller

popall

iret

The PS/2 8042 Keyboard/Auxiliary Device	Controller

On the PS/2, an	Intel 8042 chip	replaces the 8255, using ports 60H and

64H. Port 61H serves as	a system control port for compatibility	with the

PC. The	8042 controls both the keyboard	and an auxiliary device, such as

a mouse. It receives serial data, check	parity,	translates keyboard scan

codes, and presents data at the	data port 60H.	The interface can

interrupt the system (IRQ1) or can wait	for polling.  The I/O port 64H

is the command/status port. A read gives status, a write is interpreted

as a command.  The 8042	provides for a password	security mechanism.

A read from port 64H gives the following status	byte:

Bit	   Function

7	   1 = Parity error

6	   1 = General Time Out

5	   1 = Auxiliary output	buffer full

4	   1 = Inhibit switch

3	   1 = Command/data

2	   1 = System flag

1	   1 = Input buffer full

0	   1 = Output buffer full

The status register can	be read	at any time.  The data port 60H	should

be read	only when the output buffer full bit in	the status register is

1. Data	should be written to the 8042 input buffer only	when the input

buffer full bit	in the status register is 0.  If the auxiliary output

buffer full bit	is 1, then the data read came from the auxiliary device.

The command port 64H should be written to only when the	status register

input buffer full bit and the output buffer full bit are 0.  Devices

connected to the 8042 should be	disabled before	sending	a command that

generates output.

The following are recognized commands sent to port 64H:

20-3FH   Read	the 8042 RAM - Bits D5-D0 specify the address.

Address 0 is the	current	command	byte.

60-7FH   Write to the	8042 RAM   - Bits D5-D0	specify	the address.

Address 0 will mean the next byte of data out at	port 60H

is the command byte, defined using:

Bit     Function

7      Reserved = 0

6      1	= IBM keyboard translate mode

5      1	= Disable auxiliary device

4      1	= Disable keyboard

3      Reserved = 0

2      1	= Place	system flag in status register

1      1	= Enable auxiliary interrupt

0      1	= Enable keyboard interrupt

A4	   Test	if password is installed.  Data	0FAH on	port 60H means

that	the password is	installed, 0F1H	means that the password

is not installed.

A5	   Load	Security - initiate the	password load procedure.  Following

this	command	the 8042 will input from the data port until a

null	is detected.

A6	   Enable Security - enable the	security feature, when the password

pattern is currently	loaded.

A7	   Diable auxiliary device interface - set bit 5 of the	command

byte.

A8	   Enable auxiliary device interface - reset bit 5 of the command

byte.

A9	   Interface test - test the auxiliary device clock and	data lines.

The result is placed	in the output buffer at	60H:

Result	  Meaning

00	  No error

01	  Aux. device clock line stuck low

02	  Aux. device clock line stuck high

03	  Aux. device data line	stuck low

04	  Aux. device data line	stuck high

AA	   Self	test - tests 8042.  A 55H is placed in output buffer if

no errors are detected.

AB	   Interface test - cause the 8042 to test the keyboard	clock

and data lines.  Result reported as in command A9.

AC	   Reserved

AD	   Disable keyboard interface -	set bit	4 of the command byte.

AE	   Enable keyboard interface - reset bit 4 of the command byte.

C0	   Read	input port - read the 8042 input port and put it in the

output port.	 If bit	3 is 0,	the fuse on the	+5 Vdc line

on the system board to the keyboard is open.

C1	   Poll	input port low - put port 1 bits 0-3 in	status bits 4-7.

C2	   Poll	input port high	- put port 1 bits 4-7 in status	bits 4-7.

D0	   Read	output port - put data from output port	into the output

buffer.

D1	   Write output	port - put next	byte written to	60H into the

output port.	 Caution:  Bit 0 of the	output port is connected

to the System Reset line.  This bit should not be written low.

D2	   Write keyboard output buffer	- put next byte	written	to 60H

into	output buffer and issue	device interrupt if enabled.

This	produces a simulated keyboard output.

D3	   Write auxiliary device output buffer	- put next byte	written

to 60H input	buffer in output buffer	as if initiated	by the

auxiliary device and	issue interrupt	if enabled.

D4	   Write to auxiliary device - transmit	next byte written to 60H

input buffer	to auxiliary device.

E0	   Read	test inputs - cause the	8042 to	read its T0 and	T1 inputs.

This	data is	placed in the output buffer bits 0 and 1.

F0-FF	   Pulse output	port - pulse bits 0-3 of the 8042 output port

for about 6 usec.  Bits 0 to	3 indicate which bits are to be

pulsed.  A 0	indicates bit should be	pulsed.	 Caution:

Bit 0 of the	8042 output port is connected to the System Reset

line.  Pulsing this bit resets the system microprocessor.

On the PS/2, the 8042 controller can pass commands to the keyboard

through	port 60H:

ED	   Set/reset status indicators.	Rresponse is ACK (0FAH), system

acceptance of ACK requires system to	raise clock and	data lines

for at least	500 usec.

EE	   Echo	test (valid response is	EE)

EF	   Invalid command

F0	   Select alternate scan codes (response is ACK, system	then sends

option byte of 01, 02, or 03, response is ACK)

F1	   Invalid command

F2	   Read	keyboard ID (response is ACK plus two ID bytes of 83ABH)

F3	   Set typematic rate/delay (response is ACK, system sends rate/delay

byte, response is ACK.  The rate/delay byte is:

bit      function

7       reserved	= 0

6-5      (delay/250msec -	1)

4-3      doubling	factor of rate

2-0      (rate/(240/sec) - 8)

F4	   Enable (response is ACK, clears buffer, clears last typematic

key,	and starts scanning).

F5	   Default disable (resets all conditions to power-on state, sends

ACK,	stops scanning).

F6	   Set default (resets to power-on state, sends	ACK, sets default

key types for scan code 3, continues	scanning).

F7-FA	   Set all keys	to typematic, make/break, make,	or typematic/make/

break for scan code 3 (responds with	ACK).

FB-FD	   Set a key type to typematic,	make/break, or make (response is

ACK,	then keyboard prepares to receive key scan code	from set 3.

FE	   Send	the previous output again.

FF	   Reset the keyboard and start	internal self-test. Response is

ACK.	 System	must acknowledge ACK by	raising	clock and data lines

for over 500	usec.  Following acceptance of ACK, keyboard is

reinitialized and performs a	Basic Assurance	Test (BAT).  The

keyboard defaults to	scan code 2.

The PS/2 keyboard may send the following codes to the system:

00	   Key detection error/overrun under scan code 2 or 3.

83AB	   Keyboard ID bytes.

AA	   Basic Assurance Test	completed.

FC	   Basic Assurance Test	failed.

EE	   Echo	of EE command.

FA	   ACK code.

FE	   Resend. Invalid input or parity error.

FF	   Key detection error/overrun under scan code 1.

____________________________________________________________________________

----------------------------------------------------------------------------

Port 61H: PS/2 System Control Port B

----------------------------------------------------------------------------

Write operations:

Bit	 Function

7	 Reset system timer 0 output latch (IRQ0)

6	 Reserved

5	 Reserved

4	 Reserved

3	 Enable	channel	check

2	 Enable	parity check

1	 Speaker data enable

0	 System	timer 2	gate to	speaker

Read operations:

7	 1 = Parity check occurred

6	 1 = Channel check occurred

5	 System	timer 2	output

4	 Toggles with each refresh request

3	 Enable	channel	check result

2	 Enable	parity check result

1	 Speaker data enable result

0	 System	timer 2	gate to	speaker	result

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 70H and 71H: Configuration Ports and Real-Time-Clock Chip	MC146818

----------------------------------------------------------------------------

The AT and PS/2	uses port 70H bit 7 to disable 'Non-Maskable' Interrupts

(NMI) by setting bit 7 to 0.  Enable NMI by setting bit	7 to 1.	 Note:

the PCs	use port 0A0H for this purpose.	 Port 70H on ATs is also used to

set a CMOS register index (00-3FH), which is then read from port 71H.

Even when masking the NMI through bit 7	of port	70H, read port 71H

immediately after.  Otherwise, the RTC may be left in an unknown state.

The watchdog timer and system channel time-out are not masked by sending

an 80H to port 70H.

The AT stores configuration settings on	a Motorola MC146818 real time-

clock-chip (RTC).  (Programming	information for	the RTC	is given later.)

Because	the CMOS chip is supplied by a battery,	configuration parameters

are saved even during power-off. The chip has 64 registers (00-3FH) read

from port 71H after sending the	register index to 70H.	Below are some

register allocations:

Register   Use

00H	     Real-Time-Clock seconds

01H	     Real-Time-Clock seconds alarm

02H	     Real-Time-Clock minutes

03H	     Real-Time-Clock minutes alarm

04H	     Real-Time-Clock hours

05H	     Real-Time-Clock hours alarm

06H	     Real-Time-Clock day of week

08H	     Real-Time-Clock day of month

09H	     Real-Time-Clock month

09H	     Real-Time-Clock year

0AH	     Real-Time-Clock Status of register	A

(Bit 7 = 1 - time	update in progress

6-4= 22 stage	divider, clock freq. (010=32.768 KHz)

3-0= rate selection, divider output freq. (0110=1.024	KHz)

0BH	     Real-Time-Clock Status of register	B

(Bit 7 = Set update

6 = periodic	interrupt enabled

5 = alarm interrupt enabled

4 = update-ended interrupt enabled

3 = square wave enable

2 = date mode in binary (0=BCD)

1 = hours counted by	24

0 = daylight	savings	time enabled

0CH	     Real-Time-Clock Status of register	C

(Bit 7 = IRQF flag

6 = PF flag

5 = AF flag

4 = UF flag

3-0= reserved	= 0

0DH	     Real-Time-Clock Status of register	D

(Bit 7 = Valid RAM bit (0=battery	dead)

6-0= reserved	= 0

0EH	     Diagnostic	status byte

Bits   7 RTC lost	power

6 bad checksum

5 invalid configuration

4 inconsistent memory size

3 hard disk error

2 POST time check error

1-0 reserved

0FH	     Shut-down byte

10H	     Diskette drive type

Bits 7-4 first diskette	0000 = no drive

3-0 second diskette	0001 = 48 tpi drive

0010 = 96 tpi drive

11H	     Reserved

12H	     Fixed disk	drive type

Bits 7-4 first hard disk	Drive code

3-0 second hard disk	Drive code

13H	     Reserved

14H	     Peripherals (Equipment Byte)

Bits 7-6 number of diskette drives - 1

5-4 display 00=display has own BIOS

01=40 column color

10=80 column color

11=monochrome

3-2 unused

1 1=math coprocessor installed

0 0=no diskette drives, 1=diskettes installed

15H	     LSB of system board memory

16H	     MSB of system board memory

;In 1024	byte blocks, 512K increments)

17H	     LSB total expansion memory

18H	     MSB total expansion memory

;In 1024	byte blocks, 512K increments)

19H	     Drive C extension byte

1AH	     Drive D extension byte

1BH-2DH  Reserved

2EH-2FH  2 byte checksum (high, low) 10-2DH	except 0E and 0FH

30H	     LSB expansion memory above	1 megabyte

31H	     MSB expansion memory above	1 megabyte

;In 1024	byte blocks, 512K increments)

32H	     Data century byte

33H	     Information flags set during power-up

34H-3FH  Reserved

The Real-Time-Clock (RTC) on the AT and	PS/2 uses the registers

addressed at port 70H and read from 71H.  Use Int 1Ah to read and set

the time-of- day and alarm.  The alarm interrupt, Int 4Ah, must	have a

service	routine	vector before the alarm	is set.

____________________________________________________________________________

----------------------------------------------------------------------------

Port: 90H: PS/2	Central	Arbitration Register

----------------------------------------------------------------------------

Reads				  Writes

Bit	7 Enable System	Microprocessor Cycle  ESMC

6 Arbitration Mask by NMI	      Arbitration Mask

5 Bus Timeout			      Enable Extended Arbitration

4 = 0 Reserved			      =	0 Reserved

3-0 Value	of Arb.Bus During Previous    =	0 Reserved

Grant	State

____________________________________________________________________________

----------------------------------------------------------------------------

Port: 92H: PS/2	System Control Port A

----------------------------------------------------------------------------

This port supports the fixed disk drive	lights,	alternate system

microprocessor reset, PASS A20,	watchdog timer status, and CMOS

security:

Bits 7,6  Fixed disk activity	light A, B

5   Reserved = 0

4   1 =	Watchdog Timer timeout has occurred

3   1 =	RT/CMOS	secure area (password) locked by POST

2   Reserved = 0

1   Alternate Gate A20 address line (1=active)

0   Alternate CPU reset	(to effect mode	switch from

Protected Virtual Address Mode to Real Address Mode).

Reset time:	13.4 usec.  (The AT Intel 8042 method

is also supported.)

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 94H to 96H:  System Setup

----------------------------------------------------------------------------

Port 94H System Board Enable/Setup	Register

(System incluses diskette	controller, serial, and	parallel

controllers.  Set	to 0FFH	when setup is complete)

Bits  7 = 0 to setup other	system boards with I/O 100H to 107H

= 1 to avoid setup	of other system	boards

5 = 0 to setup video	subsystem with I/O 100H	to 107H

1 to avoid setup	of VGA

96H Adapter Enable/Setup Register

(Set to 00H when	setup is complete)

3 = 1 for adapter setup with	I/O 100H to 107H

0 to avoid setup	of an adaptor

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 100H-107H: PS/2 Program Option Select (POS)

----------------------------------------------------------------------------

100    PS/2 POS Reg.0 Adapter ID LSByte

101    PS/2 POS Reg.1 Adapter ID MSByte

102    PS/2 POS Reg.2 Option Select Date Byte 1

Bit 7 = Enable/Disable Parallel Poort Extended Mode

6 = Parallel Port Select high bit

5 = Parallel Port Select low bit

0	= 3BC-3BE  2 = 278-27A

1	= 378-37A  3 = reserved

4 = Enable/Disable Parallel Port

3 = Serial Port Select

2 = Enable/Disable Serial Port

1 = Enable/Disable Diskette Drive Interface

0 = Enable/Disable System Board or Card Enable

103    PS/2 POS Reg.3 Option Select Data Byte 2

104    PS/2 POS Reg.4 Option Select Data Byte 3

105    PS/2 POS Reg.5 Option Select Data Byte 4

Bit 7 = CHCK Channel Check (Set by adapter if error)

Bit 6 = STAT Channel Check Status Indicator

0 = status available at	106, 107

106    PS/2 POS Reg.6 Subaddress Extension	LSB

107    PS/2 POS Reg.7 Subaddress Extension	MSB

Only 8 bit I/O is supported on POS ports.

When video subsystem is	in setup mode (port 94H	bit 5=0), VGA responds

to a single option select byte at port 102H and	treats the bit 0 as a

sleep bit.  If bit 0 is	0, the VGA does	not respond to commands,

addresses, or data.  The VGA responds only to port 102H	when in	setup

mode. Conversely, VGA ignores address 102H when	in the enable mode (94H

bit 5=1).

____________________________________________________________________________

----------------------------------------------------------------------------

Port 0A0H: The PC NMI Mask Register

----------------------------------------------------------------------------

The 'Non-Maskable' Interrupt line to the CPU automatically generates an

Int 2 for handling disastrous situations, such as power	failure, memory

parity error, math coprocessor error, etc.  On PCs, port at 0A0H is

reserved to hold a mask	to disable this	line before it reaches the CPU.

Bit 7 set enables the NMI, while bit 7=0 disables it.  On the AT and

PS/2, use port 070H for	the same purpose (read port 71H	after to clear

the pending read state of the CMOS RAM).

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 0F0H-0FFH: 80x87 Math Coprocessor

----------------------------------------------------------------------------

Clear math coprocessor busy signal by sending 0	to port	F0H.  Reset

math coprocessor by sending 0 to port F1H.

____________________________________________________________________________

----------------------------------------------------------------------------

Port 201H: The Game Port (PC, XT, AT)

----------------------------------------------------------------------------

Port 201H contains the status of buttons 2,1 of	stick B, 2,1 of	stick A

in bits	7-4.  Bits 3-0 are set to zero by sending any byte to 201H. The

time it	takes for these	bits to	become 1 determines the	Y,X position of

stick B, Y,X position of stick A.

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 278H-27AH, 378H-37AH, 3BCH-3BEH LPT3,2,1 Printer Ports

----------------------------------------------------------------------------

These ports are	used on	the PC,	XT, AT,	and PS/2 for parallel devices.

The base port addresses	are stored in locations	40:08 to 40:0D.	 If the

address	value is zero, that port is not	available, nor are any which

follow.

The parallel ports have	the designations:

Base port:	 Data Output

Base + 1 :	 Status: Reports printer condition and errors

Bit   7 0=printer busy	(pin 11)

6 0=acknowledge data byte received (pin 10)

5 1=printer out of paper	(pin 12)

4 0=printer off line (de-select)	(pin 13)

3 0=printer error (pin 15)

2-0 unused

Base + 2 :	 Control: Initializes adapter and controls output

Bit 7-5 unused

4 1=printer interrupt enabled (IRQ status)

3 0=printer de-selected (inverted pin 17)

2 0=initialize port (delay 1/20 second after reset)

(pin	16)

1 0=no linefeed after CR	(pin 14)

0 1=output a byte of data - strobe (pin 1)

To test	if the printer is on-line, first check the existence of	the

printer	port starting at 40:08 for LPT1:.  Three parallel port words are

defined.  If a 0 value is encountered, that and	further	ports do not

exist.	Next, check the	printer	status byte, bits 3-5.	Do not begin

printing until the status register indicates that the printer is on-line

and ready to receive data.  Monitor bit	7 between each byte of data

sent.

Almost universally, Int	17h is used to control parallel	printers.  (This

is in contrast with serial devices, for	which direct chip access is

common.)

The following code shows how the printer port can be handled directly

using polling of the status byte:

MOV DX,BASE_PORT      ;	LPTx port address

LDS SI,DATA_BUFFER    ;	characters to send to printer

MOV CX,DATA_SIZE      ;	number of characters to	send

NEXT:	LODSB		      ;	get character

OUT DX,AX	      ;	send it

INC DX

INC DX		      ;	get control register

MOV AL,00001101B      ;	strobe bit set

OUT DX,AL	      ;	send strobe signal

DEC DX		      ;	get status byte

BUSY:	IN AL,DX	      ;	 into al

TEST AL,8	      ;	test for error

JZ PRT_ERROR

TEST AL,80H	      ;	check for busy

JZ BUSY

DEC DX		      ;	get data port

LOOP NEXT	      ;	continue

...

Because	of the printer,	the routine will be slow without print buffering.

An interrupt routine should be avoided with the	printer	port on	the PC

monochrome adaptor, due	to a hardware fault.  Instead, use the system

timer to determine the polling frequency.  Interrupt driven routines can

be used	on the AT and PS/2.

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 2E8-2EEH,	2F8-2FEH, 3E8-3EEH, 3F8-3FEH COM4,3,2,1

----------------------------------------------------------------------------

The 8250 Asynchronous Communication Effector (ACE) is used for control

of the serial ports on the PC.

The AT uses a NS16450, a 16-bit	version	of the 8250.  The PS/2 uses a

NS16550	which is functionally upward compatible	with the NS16450 and the

8250.

The 8250 UART (universal asynchronous receiver/transmitter) converts

parallel data on the CPU's data bus into serial data (50 to 19200 baud)

by dividing the	input clock frequency by a programmable	16-bit number.

It occupies seven sequential port addresses.  For the first serial port

on the PC, these are at	3F8-3FFH.  The second serial port occupies

2F8-2FFH.  The following internal registers are	set or read by in or out

instructions to	the corresponding port offset relative to the first

assigned port value below:

Port

Offset--> 0	   1	    2	     3	      4	       5	6

Reg.   Rec/Trans  Int	   Int	   Line	    Modem    Line     Modem

Name	 Buff	 Enable	   ID	  Contrl    Contrl  Status    Status

.--------+--------+--------+--------+--------+--------+--------.

bit  |	 data  |  rec	|  0 if	 |  word  | data   | data   | delta  |

0   |	bit 0  |  data	|pending | length |terminal| ready  |clear to|

|	       |  int	|	 | bit 0  | ready  |	    | send   |

-+--------+--------+--------+--------+--------+--------+--------|

bit  |	 data  | trans	| int id |  word  |request | over-  | delta  |

1   |	bit 1  |  data	| bit 0	 | length |to send |  run   |data set|

|	       |  int	|	 | bit 1  |	   | error  | ready  |

-+--------+--------+--------+--------+--------+--------+--------|

bit  |	 data  |  line	| int id | no. of | out	1  | parity |trailing|

2   |	bit 2  | status	| bit 1	 |stopbits|	   | error  | edge   |

|	       |  int	|	 | - 1	  |	   |	    |ring ind|

-+--------+--------+--------+--------+--------+--------+--------|

bit  |	 data  | modem	|	 | parity | out	2  | frame  | delta  |

3   |	bit 3  | status	|   0	 | enable |	   | error  |rec.line|

|	       |  int	|	 |	  |	   |	    |sig.det.|

-+--------+--------+--------+--------+--------+--------+--------|

bit  |	 data  |	|	 |  even  | loop   | break  | clear  |

4   |	bit 4  |   0	|   0	 | parity | back   | inter. |  to    |

|	       |	|	 |	  |	   |	    |  send  |

-+--------+--------+--------+--------+--------+--------+--------|

bit  |	 data  |	|	 |  stick |	   |trans.  | data   |

5   |	bit 5  |   0	|   0	 | parity |   0	   |holding |  set   |

|	       |	|	 |	  |	   |reg.empty  ready |

-+--------+--------+--------+--------+--------+--------+--------|

bit  |	 data  |	|	 |  set	  |	   |trans.  | ring   |

6   |	bit 6  |   0	|   0	 | break  |   0	   |shift   |indicat.|

|	       |	|	 |	  |	   |reg.empty	     |

-+--------+--------+--------+--------+--------+--------+--------|

bit  |	 data  |	|	 |divisor |	   |	    |rec.line|

7   |	bit 7  |   0	|   0	 | latch  |   0	   |   0    |sig.det.|

|	       |	|	 | access |	   |	    |carrier |

`--------------------------------------------------------------'

Port offset 7 (e.g. 3FFH) is not used.

If the divisor-latch-access bit	is set to 1, then the baud latch divisor

can be read or set byte	reading	or writing to port 0 (LSB) and port 1

(MSB). The clock signal	on the chip at 1.8432 MHz is divided by	the

divisor	to get the output of the baud generator	which is 16x the baud

rate.  Thus the	divisors for common baud rates are:

Baud	       Divisor Latch   Decimal

Rate		MSB	LSB	Value

300		01	80H	384

1200		00	60H	 96

2400		00	30H	 48

9600		00	0CH	 12

19200		00	06H	  6

Reset bit 7 of the line	control	register in order to access the	data and

other registers.

The Line Control Register:

The word length, number	of stop	bits, and parity are set with an out to

the line control register.  The	first two bits are defined to make a

word length using:

Bit 1	 Bit 0	Word Length

0	   0	   5

0	   1	   6

1	   0	   7

1	   1	   8

If bit 2 is 1 when bits	1 and 0	are 00,	then 1.5 stop bits are

generated. If bit 3 is 0, no parity will be used and 8 bit data	may be

transmitted. If	bit 3 is 1 and bit 4 is	0, odd parity is used. If bit 5

is 1 and bit 3 is 1, parity is transmitted as bit 4 indicates, but

received in the	opposite state.	 Setting bit 6 to 1 forces the output to

logic 0. It remains there until	bit 6 is reset.

The Line Status	Register:

Bit 0 is set 1 whenever	a byte has been	received in the	receive	buffer

register.  It is reset by reading the data or writing to this bit.

Bit 1 is set 1 if an overrun error from	the line status	register

indicates data in the receive buffer register was not read by the CPU

before another was transferred in.  It is reset	by reading the line

status register.

Bit 2 is set 1 if a parity error is detected in	the received data.  It

is reset by reading the	line status register.

Bit 3 is set 1 if a framing error is detected, i.e. the	received

character did not have a valid stop bit.

Bit 4 is set 1 if the received data is held to 0 for longer than a full

word transmission time (start bit + data bits +	parity + stop bits).

Bit 5 is set 1 if the transmitter holding register is empty.  It is

reset by loading the transmitter holding register.

Bit 6 is set 1 if the transmitter shift	register is empty.  It is reset

by data	tranfer	to it from the transmitter holding register.

Interrupt Identification Register:

Three bits are used to identify	the type of interrupt generated	by the

chip. Bit 0 is set 0 if	an interrupt is	pending.

Bits 2	and 1 are set as follows:

1	    1	if  a receive line status interrupt occurred

(overrun,	parity,	framing, or break error)

1	    0	if  a receive data interrupt occurred

(data ready to be	read)

0	    1	if  a transmitter holding empty	interrupt occurred

(trans. holding reg. just	emptied)

0	    0	if  a modem status interrupt occurred

(CTS, DSR, RI, or	Rec. Line Signal Detect-Carrier)

Modem Control Register:

The Data Terminal Ready	line can be made high by a logic 1 to bit 0 of

this register.	Similarly, a 1 in bit 1	will set the Request to	Send

line to	the modem.  The	Out 1 line is not used in the IBM PC.  However,

Out 2 must be set to 1 to enable the serial chip to interrupt the CPU

over its interrupt line.

Modem Status Register:

The modem status register indicates the	state and changes in the input

lines to the serial port.  The delta bits will be set 1	if the

corresponding lines have changed state since the last read of this

register.  The Receive Line Signal Detect line is also called the

Carrier	Detect line (RS-232 pin	8).

Sample Initialization Code:

MOV BX,3F8H	       ; com1 port

LEA DX,[BX+4]       ; use modem control port

XOR AL,AL	       ; set for all line off (dtr,rts,out1,out2)

OUT DX,AL	       ; send it

LEA DX,[BX+1]       ; use interrupt enable port

OUT DX,AL	       ; set all interrupts off

MOV AL,83H	       ; set for 8 bit,	1 sb, divisor latch

LEA DX,[BX+3]       ; get line control port

OUT DX,AL	       ; send it

MOV AX,0060H	       ; divisor for 1200 baud

MOV DX,BX	       ; lsb of	divisor	port

OUT DX,AL	       ; send it

INC DX	       ; msb of	divisor	port

XCHG	AL,AH	       ; set up	byte

OUT DX,AL	       ; send it

NOP		       ; give chip a little time

LEA DX,[BX+3]       ; line control port

IN AL,DX	       ; get back the line control byte

NOP		       ; give chip a little time

AND AL,7FH	       ; drop divisor latch

OUT DX,AL	       ; set line control byte

...		       ; set up	interrupt service routine

...		       ; and 8259 mask for int 0ch

LEA DX,[BX+4]       ; modem control register

MOV AL,0BH	       ; set for dtr,rts,out2

OUT DX,AL	       ; send it

NOP		       ; give chip a little time

LEA DX,[BX+1]       ; interrupt enable port

MOV AL,0FH	       ; use RD, TD, LS, MS interrupts

OUT DX,AL	       ; set interrupt enable register

...

(Do not	follow one OUT instruction by another on an AT or PS/2.	 Use a

delay instruction: JMP $+2 between them.  MOV AH,AL between two	OUT

instructions still does	not leave enought time for the port hardware to

recover.)

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 3B4H and 3D4H: The Motorola 6845 Video CRT Controller Address Register

Ports 3B8H-3BAH	and 3D8H-3DAH: The Motorola 6845 Video CRT Control/Status

----------------------------------------------------------------------------

The 6845 cathode ray tube controller is	used on	the PC for both

monochrome and color video systems.  The PS/2 VGA system provides some

emulation for both of these controllers	at the hardware	level.

The 6845 controller uses four I/O ports	3B4H, 3B5H, 3B8H, and 3BAH on

the monochrome card (substitute	'D' for	'B' for	color card).

Port 3B4H is the 6845 index register, to which a control register value

of 0 to	17 is sent before a read/write to the data register 3B5H.

The 6845 has 18	control	registers, 0-17.  The first ten	fix the

horizontal and vertical	display	parameters.  Incorrect settings	of

registers 1-9 can damage a monitor.  Registers 10 and 11 set the shape

of the cursor; 14 and 15 control its location.	Registers 12 and 13 can

handle scrolling. Numbers 16 and 17 report light pen position.

Registers 12-15	are read/write.	 Registers 16-17 are read only.	All

other registers	are write-only.

6845 Internal Registers:	VGA Emulation and Extension:

R0:  Horizontal	total characters	(Total characters less 5)

R1:  Horizontal	displayed characters	(Display char./line -1)

R2:  Start Horizontal blanking

R3:  End Horizontal blanking

R4:  Vertical total lines		Start Hor. Retrace Pulse

R5:  Vertical total adjust raster	End Hor. Retrace

R6:  Vertical display line		Vert. Total -2 (low 8 bits)

R7:  Vertical sync position line	Overflow  (see below)

R8:  Interlace:				Preset Row Scan	(see below)

00		  10=non-interlace

01=duplicate 11=different

R9:  Maximum raster address		Max. Scan Line (see below)

R10: Cursor Start raster

R11: Cursor End	raster

R12: Start address high

R13: Start address low

R14: Cursor high

R15: Cursor low

R16: Light pen high			Vertical Retrace Start

R17: Light pen low			Vertical Retrace End (see below)

R18:					Vertical Display Enable	End

R19:					Underline Location

R20:					Start Vertical Blank

R21:					End Vertical Blank

R22:					CRTC Mode Control

R23:					Line Compare

The VGA	emulation of the 6845 allows all registers to be read/write.

Port 3B8H is a CRT control port:

Bit	 Function	      PS/2 Emulation: None

7-6	 Reserved

5	 Blink enable

4	 Reserved

3	 Video enable

2-1	 Reserved

0	 High resolution mode

Port 3BAH is a CRT read/only status port and a write/only feature

control	port on	the PS/2.  As a	read/only port:

Bit	 Function	     PS/2 Extension: Input Status Register 1

7-6	 Reserved	     Reserved

5	 Reserved	     Attribute controller diagnostic 0

4	 Reserved	     Attribute controller diagnostic 1

3	 Video dots	     Vertical retrace

2-1	 Reserved	     Reserved

0	 Horizontal sync     Display enable (1=hor. or vert. retrace)

As a write/only	port on	the PS/2, the Write Feature Control Register,

all bits are reserved (bit 3 must be 0).

On the Hercules	Graphics Controller, bit 7 may be used to distinguish

a Hercules card	from an	IMB Monochrome or Color	Adapter.  On the

Hercules card, bit 7 goes 0 on vertical	retrace	(50 Hz).  On the

IBM card, bit 7	does not change.  The Hercules and the Hercules	Plus

can be distinguished with bits 4 and 5 (1 and 0	for Plus).

____________________________________________________________________________

----------------------------------------------------------------------------

Ports: 3C0H-3CFH: VGA Support

----------------------------------------------------------------------------

In addition to emulation for the 6845 status, index, and control ports,

the VGA	system on the PS/2 uses	the ports 3C0H-3CFH for	additional video

status information and control.

Input Status Register 1:       3BAH or 3DAH: (R)

Bit	 Function

7-6	 Reserved = 0

5-4	 Diagnostic 0,1	 Selectively connected to two of eight

3	 Vertical Retrace

2-1	 Reserved = 0

0	 Display Enable

Attribute Registers:	       3C0H-3C1H:

Attribute Controller Registers:

Bit	 Function

7-6	 Reserved = 0

5	 Palette Address Source

(Set 0 when loading color palette registers)

4-0	 Attribute Address

Each attribute data register is	written	at 3C0H	and read from 3C1H.  To

initialize the address flip-flop, issue	IOR to 3BAH or 3DAH.  Then load

the attribute controller register. This	toggles	the flip-flop for a OUT

to the indexed data register. The flip-flop is not toggled by a	read

from 3C1H.

Palette	Registers: Index 00 to 0FH:

Bit	 Function

7-6	 Reserved = 0

5-0	 P5-P0	Used to	map color input	to display color

Attribute Mode Control Register: Index 10H:

Bit	 Function

7	 P5, P4	Select 1=source	from bits 1,0 of Color Select Register

6	 PEL Width - 1 for 256-color mode

5	 PEL Panning Compatibility

4	 Reserved = 0

3	 Select	Background Intensity

2	 Enable	Line Graphics Character	Code (0=ninth dot same as backgnd)

1	 Mono Emulation

0	 Graphics/Alphanumeric Mode (1=graphics)

Overscan Color Register: Index 11H:

Bit	 Function

7-0	 P7-P0 Border color

Color Plane Enable Register: Index 12H:

Bit	 Function

7-6	 Reserved = 0

5-4	 Video Status MUX - Selects 2 of 8 color outputs for status port

3-0	 Enable	Color Plane

Horizontal PEL Panning Register: Index 13H:

Bit	 Function

7-4	 Reserved = 0

3-0	 Horizontal PEL	Panning	(number	of pixels to pan)

Color Select Register: Index 14H:

Bit	 Function

7-4	 Reserved = 0

3-2	 S_color76 - two high-order bits of 8 bit color	value

2-0	 S_color54 - replaces P5 and P4	in Attrib.Palette Reg.

Read Input Status Register 0:  3C2H: (R)

Bit	 Function

7	 CRT interrupt 1 = vertical retrace interrupt pending

6-5	 Reserved

4	 Switch	Sense Bit: Lets	POST determine monochrome or color

3-0	 Reserved

Write Misc. Output Register:   3C2H: (W)

Read  Misc. Output Register:   3CCH: (R)

Bit	 Function

7	 Vert. sync polarity 0 = positive retrace

6	 Hor. sync polarity  0 = positive retrace

bits	7,6= 1 0 for 400 lines

0 1 for 350 lines

1 1 for 480 lines

5	 Page bit for odd/even (dianostic use) 1 = high	64K page

4	 Reserved = 0

3-2	 Clock select

0 0 =	25.175 MHz for 640 hor.	pixels

0 1 =	28.322 MHz for 720 hor.	pixels

1 0 =	external clock at aux. video input (14.3-28.4 MHz)

1 1 =	reserved

1	 Enable	RAM 0 =	disable	video RAM address decode from CPU

0	 I/O address select - CRTC I/O 0 = 3BxH, 1 = 3DxH

Video Subsystem	Enable:	       3C3H:

Bit	 Feature

7-1	 Reserved

0	 Video subsystem enable: 1 = video I/O and memory address

decoding is enabled.

This register is not affected by the VGA sleep bit (102H bit 0).

Sequencer Registers:	       3C4H-3C5H:

Sequencer Address Register:    3C4H:

This register is loaded	with a index to	the following Sequence

Data registers:

Sequence Data Registers:       3C5H:

Reset Register (R/W) (Index 0):

Bit	 Function

7-2	 Reserved

1	 Synchronous reset 0 = synchr. clear and halt (before Clocking

Mode register bit	0 or Misc. Output Register bit 2)

0	 Asynchronous reset 0 =	asynchr. clear and halt

Clocking Mode Register (R/W) (Index 1):

Bit	 Function

7-6	 Reserved = 0

5	 Screen	off  1 = screen	off (use for rapid full-screen update by

giving CPU maximum memory bandwidth)

4	 Shift 4  0 = video serializers	are loaded every char. clock,

1 = video serializers	loade every fourth clock (use

with 32 bit fetches/cycle)

3	 Dot clock 0 = select normal dot clock,	1 = master clock/2

(clock/2	used for 320 and 360 hor. pixel	modes)

2	 Shift load: if	0 and if bit 4=0, video	serializers reloaded

every char. clock, when 1, every	other char. clock

(use with 16 byt	fetches/cycle)

1	 Reserved = 0

0	 8/9 dot clocks	0 = char. clocks 9 dots	wide.

Map Mask Register (R/W)	(Index 2):

Bit	 Function

7-4	 Reserved = 0

3	 Map 3 enable 1	= CPU can write	to map 3

2	 Map 2 enable

1	 Map 1 enable

0	 Map 0 enable

If this	register is set	to 0FH,	the system microprocessor can perform

32 bit wide write in only one memory cycle.

Character Map Select Register (R/W) (Index 3):

Bit	 Function

7-6	 Reserved = 0

5	 Character Map select high bit A

4	 Character Map select high bit B

3-2	 Character Map select A

1-0	 Character Map select B

In alphanumeric	modes, bit 3 of	the attribute byte normally is used

to control foreground intensity.  This bit may be redefined, however,

to switch between character sets.  For this feature to be enabled,

the following must be true:

Memory Mode register bit 1 = 1

Character Map	Select A is not	the same as Character Map Select B

If either is not true, the first 16K of	Map 2 is used.

For selection A:

Bit  5	 3   2	 Map  Table Location

0	 0   0	  0   1st 8K of	Map 2

0	 0   1	  1   3rd 8K of	Map 2

0	 1   0	  2   5th 8K of	Map 2

0	 1   1	  3   7th 8K of	Map 2

1	 0   0	  4   2nd 8K of	Map 2

1	 0   1	  5   4th 8K of	Map 2

1	 1   0	  6   6th 8K of	Map 2

1	 1   1	  7   8th 8K of	Map 2

Similarly for selection	B using	bits 4,	1, and 0.

Memory Mode Register (R/W) (Index 4)

Bit	 Function

7-4	 Reserved = 0

3	 Chain 4   0 = enable CPU to access data at addresses within

bit map using Map Mask register.

1 = enable CPU to access data at addresses according

to two low order	bits of	address	A1, A0:

00=map 0, 01=map 1, 10=map 2, 11=map 3.

2	 Odd/even  0 = use maps	0,2 or 1,3 according to	parity of address.

1 = access data sequentially	using Map Mask register

1	 Extended memory 1 = greater than 64K video memory present

0	 Reserved = 0

Digital	to Analog Converter Registers: 3C6H-3C9H:

3C6H R/W:   Pixel Mask (color	look-up	table destroyed	on write)

3C7H Read:  DAC State	Register

3C7H Write: Pixel Address

3C8H R/W:   Pixel Address

Read Feature Control Register: 3CAH: All bits reserved.

Miscellaneous Output Register: 3CCH (R)	 See port 3C2h.

Graphics Registers:	       3CEH-3CFH:

Graphics Controller Registers: 3CEH:

This read/write	register is loaded with	the index to the graphic registers

described below:

Graphics Registers (R/W)       3CFH:

Set/Reset Register (R/W) (Index	0):

Bit	 Function

7-4	 Reserved = 0

3	 Set/Reset Map 3

2	 Set/Reset Map 2

1	 Set/Reset Map 1

0	 Set/Reset Map 0

Enable Set/Reset Register (R/W)	(Index 1):

Bit	 Function

7-4	 Reserved = 0

3	 Enable	Set/Reset Map 3

2	 Enable	Set/Reset Map 2

1	 Enable	Set/Reset Map 1

0	 Enable	Set/Reset Map 0

Color Compare Register (R/W) (Index 2):

Bit	 Function

7-4	 Reserved = 0

3	 Color Compare Map 3

2	 Color Compare Map 2

1	 Color Compare Map 1

0	 Color Compare Map 0

Data Rotate Register (R/W) (Index 3):

Bit	 Function

7-5	 Reserved = 0

4-3	 Function Select  00 Data unmodified, 01 ANDed,	10 ORed, 11 XORed

2-0	 Rotate	Count for right-rotate (write mode 0)

ReadMap	Select Register	(R/W) (Index 4):

Bit	 Function

7-2	 Reserved = 0

1-0	 Map Select for	read

Graphics Mode Register (R/W) (Index 5):

Bit	 Function

7	 Reserved = 0

6	 256 color mode:

0=allow bit 5	to control loading of Shift registers

5	 Shift Register	Mode:

1=format serial data with even-numbered bits for even	maps

odd-numbered bits for odd maps

4	 Odd/Even:  1=odd/even addressing mode

3	 Read Type: 0=reads from memory	map selected by	Read Map Select	Reg.

2	 Reserved

1-0	 Write Mode for	memory map:

00=data rotated unless Set/Reset is enabled

01=from contents of system CPU latches

10=map n (0-3) filled	with 8 bits of data bit	n

11=from 8 bits in Set/Reset register for that	map

Miscellaneous Register (R/W) (Index 6):

Bit	 Function

7-4	 Reserved = 0

3-2	 Memory	Map: 00=A0000 for 128K bytes

01=A0000 for  64K bytes

10=B0000 for  32K bytes

11=B8000 for  32K bytes

1	 Odd/Even: 1=use odd/even maps for odd even addresses

0	 Graphics Mode:	1=graphics mode, 0=alphanumeric	mode

Color Don't Care Register (R/W) (Index 7):

Bit	  Function

7-4	 Reserved = 0

3	 Map 3 - Don't Care (0=Don't participate in color compare cycle)

2	 Map 2 - Don't Care

1	 Map 1 - Don't Care

0	 Map 0 - Don't Care

Bit Mask Register (R/W)	(Index 8):

Bit	 Function

7-0	 Mask: 0=bit n in each map to be immune	to change (modes 0 and 2)

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 3F0H-3F7H: Diskette Controller Ports

----------------------------------------------------------------------------

The PC uses a NEC 765 floppy disk controller.  The PS/2	uses a 8272A

diskettee controller.  The functions and port assignments for the PS/2

and AT are made	software compatible with the PC	diskette controller.

The 765	controller uses	ports 3F2H, 3F4H, and 3F5H, while the 8272 uses

ports 3F0H, 3F1H, 3F2H,	3F4H, 3F5H, and	3F7H.

Ports 3F0H, 3F1H: PS/2 Diskette	Status Registers

----------------------------------------------------------------------------

On the PS/2, the ports 3F0H and	3F1H show two of three status registers

used in	diskette operations.

Status Register	A, at 3F0H, is a read-only register showing:

Bit	 Function

7	 Interupt pending

6	 -Second drive installed

5	 Step

4	 -Track	0

3	 Head 1	select

2	 -Index

1	 -Write	protect

0	 Direction

Status Register	B, at 3F1H, is a read-only register showing:

Bit	 Function

7-6	 Reserved

5	 Drive select

4	 Write data (toggles on	positive transition in WR DATA)

3	 Read data (toggles on positive	transition in -RD DATA)

2	 Write enable

1	 Motor enable 1

0	 Motor enable 0

Ports 3F2H, 3F4H, 3F5H:	PS/2 8272 Diskette Controller:

----------------------------------------------------------------------------

Digital	Output Register, at 3F2H, is write-only, and used to control

drive motors, drive selection, and feature enable.  All	bits are cleared

by a Reset.

Bit	 Function

7-6	 Reserved

5	 Motor enable 1	when select 1 is high

4	 Motor enable 0	when select 0 is high

3	 Reserved (765 enable interrupt	and DMA	access)

2	 -8272A	Reset

1	 Reserved

0	 Drive select (0 = drive 0, 1 =	drive 1)

Diskette Drive Controller Status Register, at 3F4H, is read-only, and

used to	facilitate the transfer	of data	between	the system

microprocessor and the controller.

Bit	 Function

7	 Request for master (1 = data register ready)

6	 Data I/O direction (1 = from controller to microprocessor)

5	 Non-DMA mode if 1

4	 Diskette controller busy if 1

3-2	 Reserved

1	 Drive 1 busy (in seek mode)

0	 Drive 0 busy (in seek mode)

Data Registers for storing data, commands, parameters, and status

information, are accessed from 3F5H.

Port 3F7H is dual purpose on the PS/2:

----------------------------------------------------------------------------

Digital	Input Register at 3F7H is read-only and	used to	sense the state

of the '-diskette change' signal and the '-high density select'	signal:

Bit	 Function

7	 Diskette change

6-1	 Reserved

0	 -High density select

Configuration Control Register at 3F7H is write-only and used to

set the	transfer rate.

Bit	 Function

7-2	 Reserved

1-0	 DRC1, DRC0

00 = 500,000-bit/sec mode

01 = reserved

10 = 250,000-bit/sec mode

11 = reserved

Programming the	765 and	8272 Controllers:

The 765	and 8272 Diskette Controller performs fifteen operations,

including seek,	read, and writes.  Each	operation is performed in three

phases:	the command phase, the execution phase,	and the	result phase.

The following commands are available:

Read Data

Read Deleted Data

Read a Track

Read ID

Write Data

Write Deleted Data

Format a Track

Scan Equal

Scan Low or Equal

Scan High or Equal

Recalibrate

Sense Interrupt	Status

Specify	Step and Head Load

Sense Drive Status

Seek

As an example, the read	operation follows:

1. Turn on diskette motor and set delay time for drive	to

come up to speed.

2. Perform seek opertion.  Wait for completion	interrupt.

3. Initialize DMA chip	to move	data to	memory.

4. Send read instruction and wait for data-transfer-completion

interrupt.

5. Read status	information.

6. Turn off motor.

Operations are performed by sending a command string to	the data port

(checking the bit 6 of the status register after each byte).  Interrupt

6 is generated by the controller after a seek operation	is complete.

The interrupt handler simply sets bit 7	at 40:3EH, the seek status byte.

Poll this byte until bit 7 is set, then	reset the bit and continue with

next sector operation, initialization of the DMA chip.

____________________________________________________________________________

----------------------------------------------------------------------------

Ports 3220-3227, 3228-322E, 4220-4227, 4228-422E, 5220-5227, 5228-522E:

----------------------------------------------------------------------------

These are the assigned COM3-8 serial ports on the PS/2,	all utilizing

IRQ3 interrupt line.  For programming information, see ports 2E8-2EE.

____________________________________________________________________________

----------------------------------------------------------------------------