#

File: canon.info,  Node: Top, Prev: (akai.info), Next: (denon.info),  Up: (remotes.info)

******************************************************************************

CANON

******************************************************************************

* Menu:

* Technical info::

* Videocamera::

#

File: canon.info,  Node: Technical info,  Prev: Top,  Next: Videocamera,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            32 bits

Carrier:                37.5kHz

+--------------------------------+

Header:    |                                |

+                                +----------------+..

16T  (8800us)            8T (4400us)

+--+

1 is coded:  |  |

+  +------+..

T    3T

+--+

0 is coded:  |  |

..+  +--+..

T  T

T = 550us

+--+

|  |

After the last bit a ( ..+  +  ) is transmitted to terminate the last bit.

T

Space between transmissions = 40ms

#

File: canon.info,  Node: Videocamera,  Prev: Technical info,  Up: Top

******************************************************************************

VIDEOCAMERA

******************************************************************************

GROUP   #A16E

Code    Function

----    --------

#C03F   Start/Stop

#B847   Zoom wide

#387C   Zoom tele

#807F   Play

#E817   Stop

#40BF   Rew

#00FF   FF

#20DB   Pause/Still

#E01F   REC

#708F   Char title

#F00F   Date

#B04F   Count reset

#609F   Tape return

#D827   On Screen