WINBOND  EMS 4.0 ( version 1.10 ) USER MANUAL

1. The EMS 4.0 driver is loaded by the CONFIG.SYS at booting time .

2. The default values of the EMS configuration registers are set at the

below :

The BASE FRAME ADDRESS = 0D000H

The I/O PAGE REGISTER ADDRESS = 208H

The total of HANDLE NUMBER = 256

3. The EMS 4.0 driver command line has to be like the following :

DEVICE = WINEMM2.SYS                   (use default value)

or

DEVICE = WINEMM2.SYS /Pxxx /Iy /Mz /A

or

DEVICE = WINEMM2.SYS -Pxxx -Iy -Mz -A

where

P - handle numbers	     xxx : 64 - 256 ( by decimal )

I - page register address      y : 0 - F ( by hexidecimal )

y = 0  -->  208h/209h , 4208h/4209h , 8208h/8209h , c208h/c209h

1  -->  218h/219h , 4218h/4219h , 8218h/8219h , c218h/c219h

2  -->  228h/229h , 4228h/4229h , 8228h/8229h , c228h/c229h

3  -->  238h/239h , 4238h/4239h , 8238h/8239h , c238h/c239h

4  -->  248h/249h , 4248h/4249h , 8248h/8249h , c248h/c249h

5  -->  258h/259h , 4258h/4259h , 8258h/8259h , c258h/c259h

6  -->  268h/269h , 4268h/4269h , 8268h/8269h , c268h/c269h

7  -->  278h/279h , 4278h/4279h , 8278h/8279h , c278h/c279h

8  -->  288h/289h , 4288h/4289h , 8288h/8289h , c288h/c289h

9  -->  298h/299h , 4298h/4299h , 8298h/8299h , c298h/c299h

a  -->  2a8h/2a9h , 42a8h/42a9h , 82a8h/82a9h , c2a8h/c2a9h

b  -->  2b8h/2b9h , 42b8h/42b9h , 82b8h/82b9h , c2b8h/c2b9h

c  -->  2c8h/2c9h , 42c8h/42c9h , 82c8h/82c9h , c2c8h/c2c9h

d  -->  2d8h/2d9h , 42d8h/42d9h , 82d8h/82d9h , c2d8h/c2d9h

e  -->  2e8h/2e9h , 42e8h/42e9h , 82e8h/82e9h , c2e8h/c2e9h

f  -->  2f8h/2f9h , 42f8h/42f9h , 82f8h/82f9h , c2f8h/c2f9h

M - base memory address	     z : 0 - 8 ( by decimal )

z = 0  -->  c000h , c400h , c800h , cc00h

1  -->  c400h , c800h , cc00h , d000h

2  -->  c800h , cc00h , d000h , d400h

3  -->  cc00h , d000h , d400h , d800h

4  -->  d000h , d400h , d800h , dc00h

5  -->  d400h , d800h , dc00h , e000h

6  -->  d800h , dc00h , e000h , e400h

7  -->  dc00h , e000h , e400h , e800h

8  -->  e000h , e400h , e800h , ec00h

* The command line parameters can be in either upper or lower case

* If the base memory address conflicts with the SYSTEM ROM address , it

must be changed another value .

* Don't put any extra leading zero in the parameter.

DEVICE = WINEMM2.SYS /P256 /I0  /M1  /A  (correct)

DEVICE = WINEMM2.SYS /P256 /I00 /M01 /A  (error,never use it)

The extra zero (/I00,it should be /I0) in the above line will cause

some trouble while installing the EMS driver.