���������������������Ŀ

�Library Documentation�

�����������������������

Library name: sb

������������

Library description : Sound Blaster Card functions.

�������������������

Library notes :  Sound Cards supported :

������������-       Sound Blaster 1.0-2.0/Pro/Pro 2/16/AWE 32 or compatible.

Base port : 0x200-0x280

IRQ : 2-11

DMA : 0,1,3,5,6,7

Playing capabilities (not influenced by Sound Card type):

8 bit/16 bit

signed/unsigned

mono/stereo/surround

4..44Khz

256 different samples in the same time

Libraries used : cmdline,mem.

��������������

Example(s) : examples\sbdetect.xss

����������   examples\sbplay.xss.

Library structures

������������������

SBCARD

������

Sound Blaster Card description.

Library variables

�����������������

word port

���������

Sound Blaster base port address (0x200-0x280).

byte type

���������

Sound Blaster type : 1-1.0

2-2.0

3-Pro

4-Pro 2

5-16

6-AWE32

byte lover,hiver

����������������

Sound Blaster DSP type.

byte irqno

����������

Sound Blaster IRQ number.

byte dma

��������

DMA channel used by Sound Blaster.

byte dma16

����������

DMA channel used by Sound Blaster 16.

char *name

����������

Sound Blaster name.

dword flags

�����������

Sound blaster capabilities flags.

bit 3  stereo

bit 5  16 bit

dword maxmonofreq

�����������������

Maximum frequency supported by Sound Blaster Card on mono

playing.

dword maxstereofreq

�������������������

Maximum frequency supported by Sound Blaster Card on

stereo playing.

SBCFG

�����

Sound Blaster Card playing configuration.

Structure variables

�������������������

dword freq

����������

Playing frequncy.

dword playflags

���������������

Playing flags.

bit 3   stereo

bit 5   16bit

bit 6   surround

bit 7   invert L/R channels

byte  globalvol

���������������

Global Sound Card volume (0 to 255).

byte res

��������

Reserved (0).

byte leftvol

������������

Left channel volume (0 to 255).

byte rightvol

�������������

Right channel volume (0 to 255).

byte trebleleft

���������������

Left channel treble value (0 to 255).

byte trebleright

����������������

Right channel treble value (0 to 255).

byte bassleft

�������������

Left channel bass value (0 to 255).

byte bassright

��������������

Right channel bass value (0 to 255).

SBSMP

�����

Sound Card playing sample structure.

This structure is used by playing functions.

Structure variables

�������������������

dword type

����������

Reserved.

dword flags

�����������

Sample playing flags :

bit 2   loop

bit 3   stereo

bit 4   signed

bit 5   16 bit

bit 8   paused

Other bits are reserved and must be 0.

dword info0,info1,info2,info3

�����������������������������

Reserved, do not modify.

byte *buf

���������

Sample buffer.

dword size

����������

Sample size.

dword freq

����������

Sample frequency.

dword pos

���������

Sample position.

byte  globalvol

���������������

Global sample volume (used if sample is mono).

Valid values : 0 to 255.

byte res

��������

Reserved, must be 0.

byte leftvol

������������

Left channel sample volume (used if sample is stereo).

Valid values : 0 to 255.

byte rightvol

�������������

Right channel sample volume (used if sample is stereo).

Valid values : 0 to 255.

dword info9

�����������

Reserved, must be 0.

dword vid[4]

������������

Reserved, must be 0.

Library functions

�����������������

SBCARD *_sb_getcard()

���������������������

Returns a pointer to the current Sound Card structure.

If no sound card has been set, returns 0.

dword _sb_setcard(SBCARD *card)

�������������������������������

Sets the specified Sound Card configuration and tries to initialise

it.

Returns an error code (see _sb_geterrorstr).

dword _sb_autodetect()

����������������������

Tries to auto-detect the Sound Blaster Card.

Returns an error code (see _sb_geterrorstr).

SBCARD *_sb_getenvcard();

�������������������������

Returns the Sound Blaster Card configuration from the environment.

Returns 0 on failure.

This function doesn't update the current Sound Card configuration.

SBCFG *_sb_getcfg()

�������������������

Returns the current Sound Blaster playing configuration.

Returns 0 if no sound card has been detected.

dword _sb_setcfg(SBCFG *cfg)

����������������������������

Sets the specified Sound Blaster playing configuration.

Returns an error code (see _sb_geterrorstr).

dword _sb_switchsurround()

��������������������������

Switch the surround bit from the Sound Blaster playing flags.

Returns an error code (see _sb_geterrorstr).

dword _sb_invertchannels()

��������������������������

Switch the invert L/R channels bit from the Sound Blaster playing

flags.

Returns an error code (see _sb_geterrorstr).

dword _sb_startplaying()

������������������������

Starts playing.

Returns an error code (see _sb_geterrorstr).

dword _sb_stopplaying()

�����������������������

Stops playing.

Returns an error code (see _sb_geterrorstr).

dword _sb_switchplaying()

�������������������������

If SB is playing, stops it. Else, start playing.

Returns the playing status (0=OFF,1=ON).

SBSMP *_sb_makesmp(byte *buf,dword size,dword freq,dword flags)

���������������������������������������������������������������

Makes a sample structure with the specified information and

returns a pointer to it.

If error (not enough memory), returns 0.

SBSMP *_sb_dupsmp(SBSMP *smp)

�����������������������������

Duplicates a sample structure.

If error (invalid sample structure), returns 0.

dword _sb_freesmp(SBSMP *smp)

�����������������������������

Frees a sample structure.

Returns an error code (see _sb_geterrorstr).

dword _sb_playsmp(SBSMP *smp)

�����������������������������

Starts playing the specified sample.

Returns an error code (see _sb_geterrorstr).

dword _sb_stopsmp(SBSMP *smp)

�����������������������������

Stops playing the specified sample.

Returns an error code (see _sb_geterrorstr).

dword _sb_stopallsmp()

����������������������

Stops playing all samples.

Returns an error code (see _sb_geterrorstr).

dword _sb_pausesmp(SBSMP *smp)

������������������������������

Pause playing specified sample.

Returns an error code (see _sb_geterrorstr).

dword _sb_resumesmp(SBSMP *smp)

�������������������������������

Resume playing specified sample.

Returns an error code (see _sb_geterrorstr).

dword _sb_updatesmpvol(SBSMP *smp)

����������������������������������

Changes the specified sample volume.

Returns an error code (see _sb_geterrorstr).

dword _sb_enablespk()

���������������������

Enable Sound Blaster speaker output.

Returns an error code (see _sb_geterrorstr).

dword _sb_disablespk()

����������������������

Disable Sound Blaster speaker output.

Returns an error code (see _sb_geterrorstr).

char *_sb_geterrorstr(dword errno)

����������������������������������

Returns a pointer to a string associated to the specified error

code.

Errors codes : 0 - no error

1 - no sound card detected (yet)

2 - no stereo sound card

3 - no 16 bit sound card

4 - could not detect SB port

5 - could not detect SB IRQ

6 - could not detect SB DMA

7 - sound card in use

8 - invalid sample structure

9 - sample is not playing

10 - could not start playing

11 - not enough low memory for playing buffer

12 - sound card not playing

13 - sound card not paused