TERMZ100 INSTRUCTIONS

This utility is a comprehensive terminal program for the Z-100 on

the  order of the CPS program for the Z-90.   It turns the  Z-100

into  a  terminal of choice for use in access to  other  computer

systems.   Keyboard expansion is disabled and all key entries are

converted to a desired character or character sequence (e.g.  the

up-arrow  key is usually converted to ESC,A in most ASCII  termi-

nals).   In this manner, the keyboard may be made to emulate most

any terminal on the market by providing the proper entries in the

keyboard  conversion table.   Incomming data from the modem  port

also  passes  through  a filter such that the  emulated  brand  X

terminal's  control and escape sequences are translated to  those

recognized by the Z-100.   The table entries provided in TERMZ100

are for emulation of the Z-100 itself.

Once  activated,  the program is in its on-line mode which  means

that  any characters typed on the keyboard are converted and sent

to the AUX port.   Any characters received from the AUX port  are

converted if necessary and displayed directly on the CRT.   Also,

characters both sent and received to/from the AUX port are stored

in memory for later retrieval if desired.

Certain  capabilities  are also available to provide some  local

services within the terminal program.   Three keys are  monitored

for  entry  on the keyboard and are not passed to the  AUX  port.

They are:

Control/P  - Causes the current contents of the CRT screen to  be

saved  in memory and then printed on the PRN device.   As soon as

the cursor returns,  the keyboard/display is free for further use

while the printer is busy printing (transparent print).

Shift/Break  - A  form feed is saved in the memory buffer  and  a

special character is displayed.  Nothing is sent to the AUX port.

This is to allow for placement of page breaks in the saved record

of a terminal session for later display on a printer.   If trans-

parent  print  is active,  a page break is sent  to  the  printer

directly.

Help - The contents of the screen are saved and a menu of options

is displayed.   While the Help menu is being displayed,  only the

following keys are active and they have the following meaning:

F1  - Return  On-Line  - The saved screen is redisplayed  at  the

point at which the Help key was pressed and the program is  again

in its on-line mode as described above.

F2 - Option Toggles - The user is prompted for one of the options

which may be toggled on/off:

1  - 25th  status line on/off - The 25th line  will  contain

(when  enabled)  an  event timer and  a  memory  utilization

percentage.   These  data are useful when using this program

for connection to a data-base service or other timed connec-

tion.  When using this program with systems which themselves

make  use of the 25th line (e.g.   protocol converters)  the

local  use of the status line must be turned off.   The  de-

fault state of the status line is off.

2 - Character echo on/off - Characters typed on the keyboard

when  in  on-line mode are echoed to the CRT with  the  echo

flag on (default state is off).  This flag should be on when

communicating with systems which do not echo received  char-

acters (e.g.  another Z-100 system using this program or one

like it).  It should be off when communicating with a system

in  FDX mode (e.g.  a protocol converter or a data base ser-

vice).

3 - Transparent print on/off - When this flag is on (default

is off), all characters received are also transferred to the

printer as well as to the CRT and memory buffer.  This is to

allow for simultaneous hard copy of a conversation.

F3 - Change Current File - The user is prompted for a new  active

file  name  (note:  the current active file is displayed  at  the

bottom  of the Help menu screen).   Upon entry,  that new file is

opened and becomes the new active file.   A null entry will cause

the  retention  of the old active file.   This file  is  used  by

options F5, F6, and F7 below.

F4  - Clear  Counters  - The user is prompted to  clear  (1)  the

memory  buffer which contains an ongoing record of all communica-

tions  activity or (2) the event timer which is a record of  con-

nect time.

F5  - Save  Memory Buffer - The current contents  of  the  memory

buffer are written to the currently active file.

F6  - Transmit  Current File (See Note 1) - The user is  prompted

for the type of file transmission desired:

1 - ASCII file to Z-100 w/ TERMZ100 protocol

2 - Binary file to Z-100 w/ TERMZ100 protocol

3 - Direct ASCII file transfer (no protocol)

F7  - Receive Current File - This is an error  checked  reception

only  (see note 1).   To receive direct transfers (see F6  option

3), use the memory save option F5.

F8  - Change  AUX Baud Rate - The user is prompted to change  the

baud rate to:

1 - 300 bps

2 - 1200 bps

3 - 2400 bps

4 - 4800 bps

5 - 9600 bps

The AUX port should be set through CONFIGUR to the proper parity,

etc.  desired.   For  file transfer between  Z-100  systems,  the

parity  on both systems must match.   It is reccommended that the

AUX port be setup with 7-bit characters, even parity, 1 stop bit.

The  4800 and 9600 bps options should only be used  with  systems

supporting  XON/XOFF.   The  input  buffer for the AUX  port  was

short-sightedly fixed at 120 characters in the current version of

Z-DOS.  This coupled with the fact that the fastest the Z-100 can

process  characters to its CRT is only about 4800 bps forces  the

use of XON/XOFF to prevent buffer overflows which would otherwise

occur (assuming the other end can respond to XOFF fast enough).

F9  - Save  Program  Image - This option will  save  the  current

program  image  on the default drive in  the  file  TERMZ100.COM.

This  will  make permanent any set baud rate,  active  file,  and

programmed function key strings.

F10  - Set Programmed Function Keys - Function keys F1-F8 can  be

programmed by the user.   The current values for these 8 function

keys  are  displayed and the user is prompted for the key  to  be

modified.   The user is then asked to enter the character  string

to  be associated with that key (63 characters max) terminated by

control/Z.   Current key values are then re-displayed.   To  exit

this  mode,  type any character other than 1-8.   In on-line mode

the  programmed function keys are expanded to their stored  value

and sent to the AUX port.   Unprogrammed keys will cause transfer

of  the escape sequence normally associated with that  particular

function key.   To de-program a key,  enter a null string for the

function key value (control/Z only).

F11 - Print Memory Buffer - The contents of the memory buffer are

sent to the PRN device.   The print may be aborted at any time by

pressing any key on the keyboard.

F12  -  Exit Terminal Program  -  Return to Z-DOS.

Note  1:   The file transfer facility in options F6 and F7  allow

for error corrected file transfer between Z-100 systems,  each of

which  is  running this program.   The transfer  protocol  is  as

follows:

When started by an operator,  the transmitting computer will send

either  an SOH for binary file transfer or an STX for ASCII  file

transfer  and will await an ACK response from the receiver.   The

transmitter will send the SOH/STX request every 3 seconds for  30

seconds  waiting  for the receiver's response before  giving  up.

The  receiver,  when  started by an operator,  will wait  for  30

seconds  for  an  SOH/STX transmit request  from  a  transmitting

computer before it will give up.

When  the receiver gets the SOH/STX request,  an ACK response  is

sent.   The  transmitter will then send data in 256-byte  blocks.

Blocks  are  terminated with ETB/LRC for intermediate blocks  and

ETX/LRC  for the final or only block.   The receiver will  verify

the received block and respond with ACK or NAK.   The transmitter

will  retransmit NAKked blocks for a maximum of 10  times  before

aborting the transmission.

Transmission  may be terminated by either end by transmitting  an

EOT  character.   An operator can force a transmission abort from

either end by typing control/A on the keyboard.   All other  keys

will be ignored during file transmission.

The difference between binary and ASCII file transmission is that

for  binary  files a 2 for 1 character expansion is performed  by

the transmitter prior to block transmission and a 1 for 2 charac-

ter compression is performed by the receiver after recep-tion  of

a block.   This is to prevent control characters in the data from

screwing up the communications protocol.  In ASCII file transfer,

the  contents  of  the ASCII file are transmitted on an  "as  is"

basis.   Only the least significant 7 bits of the 8-bit byte  are

transferred.   Any control characters (SOH,  STX,  ETX, EOT, ETB)

which  may be in the data file will consequently cause communica-

tions errors.   Files which may not be transferred in ASCII  form

are COM files,  EXE files,  Multiplan work files,  Basic programs

stored in non-ASCII form, etc.  Files which may be transferred in

ASCII are source files, print files, etc.

This  program is written is Assembler and requires 64K of  memory

in which to operate.

Comment:  The  method of error corrected file transfer is a  very

primative "roll-my-own", having no documentation on other methods

short of full bi-sync 2780/3780 implemented in ASCII async (not a

bad  idea,  but this program is large enough as it is).   A  good

improvement  to this program would be to change the file transfer

method to that used by HUG (on which I have no information).

This  program  would be much more efficient in its  emulation  of

terminals  if it could bypass the Z-100 ROM keyboard/CRT  proces-

sing.   Having  no documentation on this ROM,  I was  stuck  with

using it as is.

#OM keyboard/CRT