MICROSOFT MOUSE README.DOC

This document contains information that became available

after the "Microsoft Mouse User's Guide" was printed.

MICROSOFT MOUSE HARDWARE NOTES

"Microsoft Mouse: Driver Not Installed -- Mouse Not Found"

Error Message

If you receive the message "Microsoft Mouse: Driver Not

Installed -- Mouse Not Found" after you have connected a

Serial mouse to your computer and have run MSETUP to install

the mouse software, it may indicate a serial port

configuration problem. This is particularly true when

multiple serial adapters or a serial adapter and a modem are

installed in the system. Check the configuration of each

serial port as specified in the manufacturer's installation

guide. Be sure that non-IBM serial adapters are configured

to be 100% IBM compatible. If you receive this "Microsoft

Mouse Driver Not Installed" message and cannot find any

problems with your system's serial port configuration, or you

do not have the mouse attached to a serial port, it may indicate

a mouse hardware problem and you should contact the dealer

from whom you purchased your mouse.

MICROSOFT MOUSE SOFTWARE NOTES

Microsoft Mouse Drivers -- Older Versions

MSETUP will delete any line containing "CPANEL" in the

AUTOEXEC.BAT file (unless the line starts with "SET," "PATH,"

or "REM").

Single-drive PCs

If you have a single-drive PC without a hard disk, you will

not be able to use the Mouse Setup program (MSETUP) to

set up the mouse software.

MSETUP installs a mouse driver, MOUSE.COM. (For more

information, see "What the MSETUP Program Does" in  of the

"Microsoft Mouse User's Guide.") On a single-drive PC, you

must install the mouse driver yourself in one of two ways:

Manual Installation -- Copy MOUSE.COM onto the disk you use

to start your computer. You can then load the mouse driver

manually each time you start your system, by typing "mouse"

at the DOS prompt.

Automatic Installation -- With MOUSE.COM on the disk you use

to start your computer, you can modify your AUTOEXEC.BAT file

so that the mouse driver is loaded automatically each time you

start your system. Add the line "MOUSE" to the AUTOEXEC.BAT file

to load MOUSE.COM automatically when you start your computer.

Invoking the Control Panel In Microsoft Word

Microsoft Word recognizes the Ctrl-left mouse button and

Shift-left mouse button key combinations as Word operations.

Therefore, in order to invoke the Control Panel while

working in Word, you need to hold down the Ctrl key and then

press the Alt or Shift keys before pressing the left mouse

button.

Using the Mouse with Microsoft Windows Version 1.01

If you are using Microsoft Windows version 1.01, you must

replace the MOUSE.DRV file on the Windows Setup disk with

the MOUSE.DRV file located on the Microsoft Mouse Setup

disk in this package and then reinstall Windows. This

procedure only takes a few minutes and does not destroy

existing data in your Windows subdirectory. Be sure that

you reinstall Windows in the same subdirectory you used the

first time you installed it. For more information on

installing Windows, see the "Microsoft Windows User's Guide."

Note: If you are using Windows version 1.03 or later, you do

NOT have to replace the MOUSE.DRV file on the Windows Setup

disk.

ADDITIONAL INFORMATION

Additional information regarding the application included in

your Microsoft Mouse package may have been included in a

readme file. Please check the application disk for a readme file.